Front cover image is taken at Bibra Lake Regional Adventure Playground, Bibra Lake.

| Document Control |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Version No | Date | Version Details | Author | Reviewer | Approver |
| 1 | Jan 2013 | Draft to managers | Asset Services | Park Services |  |
| 2 | Feb 2013 | Final Version for adoption | Asset Services | Manager Parks & Environment | Council |
| 3 | Aug 2015 | Final Version 2014-2017 | Asset Services | Manager Parks & Environment | Executive Group |
| 4 | June 2018 | Final Version 2017-2020 | Asset Services | Manager Parks & Environment | Manager Parks & Environment |
| 5 | Sept 2021 | Final Version 2020 - 2024 | Property & Assets | Manager Environment, Parks and Landscape | Executive Committee (ExCo) |

Acknowledgement of Country

The Mayor, Councillors and staff of the City of Cockburn acknowledge the Whadjuk Nyungar people of Beeliar boodja as the traditional custodians of this land. We pay our respect to the Elders, past, present and emerging.

Table of Contents

Glossary	ii

1.	Executive summary	1

2.	Introduction	5

2.1	Background	5

2.2	Goals and Objectives of Asset Management	8

2.3 Plan Framework	10

2.4 Asset Management Maturity	10

2.5 Asset Management Plan – Data confidence assessment	12

3. Levels of Service	13

3.1 Customer Research and Expectations	13

3.2 Current Levels of Service	15

3.3 Enterprise Risk Management	25

3.4 Legislative Requirements	27

3.5 Asset Capacity and Performance	27

4. Future Growth And Demand	29

4.1	Growth Forecast	29

4.2	Changes in Technology	31

4.2.1 Alternate Water Supply System (AWSS) Groundwater Interception Device (GID)	32

4.2.2 Fertigation	34

4.2.3 Soil Moisture Sensors (SMS)	34

4.3	Demand Management Plan	34

4.4	New Assets from Growth	36

5. Lifecycle Management Plan	37

5.1 Asset Data	37

5.1.1 Asset Condition	38

5.1.2 Useful life	41

5.1.3 Asset valuations	44

5.2 Maintenance and Operating expenditure	46

5.2.1 Standards and specifications	51

5.2.2 Summary of future maintenance expenditures	52

5.3 Renewal/Replacement Plan	53

5.3.1 Playgrounds	53

5.3.2 Irrigation	53

5.3.3 Infrastructure	54

5.3.4 Renewal standards	55

5.3.5 Summary of future renewal expenditure	55

5.4 New/upgrade plan	55

5.4.1 Selection criteria	56

5.4.2 Summary of future upgrade/new assets expenditure	56

5.5 Disposal Plan	56

6. Financial Analysis	57

6.1	Financial Statements and Projections	57

6.2 Funding Strategy	61

6.3 Sustainability of Service Delivery	61

6.3.1 Asset Consumption Ratio	62

6.3.2 Asset Sustainability Ratio	62

6.3.3 Asset Renewal Funding Ratio	63

6.4 Valuation Forecasts	64

6.5 Key Assumptions made in Financial Forecasts	66

7.	Asset Management Practices	67

7.1	Accounting/Financial Systems	67

7.2 Asset Management Systems (EAM)	67

7.3	Information Flow Requirements and Processes	68

7.4	Standards and Guidelines	68

8.	Plan Improvement And Monitoring	69

8.1	Performance Measures	69

8.2	Improvement Strategy	69

8.3	Monitoring and Review Procedures	71

References	73

Appendices	74

Appendix A	Legislative requirements	74

Appendix B	Major Park & Environment New Capital Works Projects	76

Appendix C	Maintenance Response Levels of Service	80

Appendix D	Preliminary10 Year Playground Renewal Program	82

Appendix E	Preliminary 6 Year Irrigation Renewal Program	86

Appendix F   Projected 10 Year Infrastructure Renewal Program	88

# Glossary

ASPEC (M, O, R, D) Specification

ASPEC data Specification and the City’s operational register classification i.e. Marina and Coastal Infrastructure, Open Space, Road and Drainage Specification.

Asset

A physical component of a facility which has value enables a service to be provided and has an economic life of greater than 12 months.

Asset Class

Groupings of assets of similar nature and use in a local government’s operations (AASB 166.37)

Asset Classification

A division of the asset class regarded as having particular shared characteristics

Asset Type

Defines the range of assets held in the asset classification ie. ASpec

Asset Condition

Is a measure of the asset’s physical integrity to enable prediction of maintenance, rehabilitation and renewal requirements.

Asset Management

The combination of management, financial, economic, engineering and other practices applied to physical assets with the objective of providing the required level of service in the most cost effective manner.

Capital Renewal Expenditure

Expenditure/ works on an existing asset which returns the service potential or the life of the asset to that which it had originally.

Capital New Expenditure

Expenditure used to create new assets or to increase the capacity of existing assets beyond their original design capacity or service potential.

Capital Upgrade Expenditure

Expenditure which enhances an existing asset to provide a higher level of service or expenditure that will increase the life of the asset beyond that which it had originally.

Current Replacement Cost (CRC)

The cost of replacing the service potential of an existing asset, by reference to some measure of capacity, with an appropriate equivalent asset.

Depreciation

The wearing out, consumption or other loss of value of an asset whether arising from use, passing of time or obsolescence through technological and market changes.

*The systematic allocation of the depreciable amount (service potential) of an asset over its useful life.

Depreciated Replacement Cost

The replacement cost of an existing asset less an allowance for wear and consumption, having regard for the remaining economic life of the existing asset.

Expenditure

The spending of money on goods and services.

Fair value

Fair value is defined as the price that would be received to sell an asset or paid to transfer a liability in an orderly transaction between market participants at the measurement date.

Funding gap *

Difference between estimated budgets and projected expenditures from the Long Term Financial Plan for maintenance and renewal of assets, totalled over a defined time.

Gap Analysis

A method of assessing the gap between a business’s current asset management practices and the future desirable asset management practices.

Integrated Planning and Reporting
A framework for establishing community priorities and linking this information into different parts of a local government’s functions.

Level of service *

The defined service quality for a particular activity or service area against which service performance can be measured. Service levels usually relate to quality, quantity, reliability, responsiveness, environmental, acceptability and cost.

Life Cycle Management

The total cost of an asset throughout its life including costs for planning, design, construction, acquisition, operation, maintenance, rehabilitation and disposal.

Long Term Financial Plan (LTFP)
Supported by the Asset Management Planning Process the LTFP is a ten year rolling plan that informs the Corporate Business Plan to activate Strategic Community Plan priorities. From these planning processes, Annual Budgets that are aligned with strategic objectives can be developed.

Maintenance

All actions necessary for retaining as asset as near as practicable to its original condition, but excluding rehabilitation or renewal.

Non-Asset Solution

The process used to identify the alternative methods of addressing, reducing and/ or increasing demand for services other than by adjusting asset capacity.

Operating expenditure *

Recurrent expenditure, which is continuously required excluding maintenance and depreciation, eg power, fuel, staff, plant equipment, on-costs and overheads.

Planned Maintenance *

Repair work that is identified and managed through a maintenance management system, activities include inspection, assessing the condition against failure/breakdown criteria/experience, prioritising scheduling, actioning the work and reporting what was done to develop a maintenance history and improve maintenance and service delivery performance.

Reactive maintenance *

Unplanned repair work that is carried out in response to service requests and management/supervisory directions.

Remaining life *
The time remaining until an asset ceases to provide the required service level or economic usefulness.  Age plus remaining life is economic life.

Replacement Cost

The cost of replacing an existing asset with a substantially identical new asset.

Risk management *

The application of a formal process to determine the range of possible values relating to key factors associated with a risk in order to determine the resultant ranges of outcomes and their probable occurrence.

Strategic Community Plan
The strategy and planning document that reflects the longer term (10+ year) community and local government aspirations and priorities.

Useful life *

Either:

(a) the period over which an asset is expected to be available for used; or

(b) the number of production or similar units (i.e. intervals, cycles) that is expected to be obtained from the asset..

Source: Government of WA Asset management framework and guidelines, Glossary

*Source:  DVC 2006, Glossary ‘Asset Investment Guidelines’

# 1.	Executive Summary

With the implementation of the City’s Integrated Corporate planning Framework, the Parks & Environment Asset Management Plan (PEAMP) has been developed to establish sustainable financial management, robust governance, continuous improvement and best practice management of the City’s infrastructure assets.

The PEAMP covers the 2020-21 to 2023-24 financial years and outlines the services provided by the Parks and Environment Service Unit in delivering strategic and operational asset management activities for communities that utilise Parks and Environment assets.

The PEAMP is one of eight infrastructure AMPs developed by the City and forms part of the City’s Strategic Asset Management Planning Framework. The PEAMP will be developed every four years in alignment with the Corporate Planning Framework ensuring that the City’s long term financial planning (LTFP) is supported by timely and accurate asset information and financial projections derived from a structured and strategic asset management planning process.

The Parks & Environment Asset Management Plan (PEAMP) covers Irrigation, Park Infrastructure and Playgrounds. The data utilised in the creation of the Parks & Environment Asset Management Plan is based on the City’s operational asset register which is considered to be approximately 95% accurate.  The condition ratings were last assessed in 2019 following a comprehensive data pick up survey.

The City’s previous PEAMP has been reviewed and updated to present a 2020 version that reflects the current status of Parks & Environment assets for future asset management planning. The plan demonstrates the current Asset Management maturity and further reinforces the City’s commitment to continuously improve its asset management practice and methodologies.

Table 1.1 PEAMP Infrastructure Assets Summary Table as at November 2020

| Asset Classification | 2013-14 | 2014-15 | 2016-17 | 2019-20 |
| --- | --- | --- | --- | --- |
|  | Replacement value | Replacement value | Replacement value | Replacement value |
| Amenities | $4.45m | $4.05m | $2.15m | $7.89m |
| Bins | $0.1m | $0.1m | $0.2m | $0.7m |
| Fences | $6.81m | $7.5m | $12.99m | $35m |
| Irrigation | $18.98m | $17.9m | $19.65m | $18.14m |
| Lighting | $3.74m | $3.4m | $3.7m | $6.47m |
| Minor structure | $8.23m | $7.1m | $9.98m | $14.91m |
| Playgrounds | $15.18m | $14.0m | $17.5m | $22.1m |
| Public Art | - | - | $1.5m | $1.5m |
| Signs | $0.9m | $1.0m | $1.03m | $1.55m |
| Sporting Equipment | - | - | $0.6m | $2.4m |
| TOTAL | $58.4m | $55.1m | $69.3m | $110.6m |

In September 2020 the revaluation carried out in consultation with Finance Services reviewed rates for parks hard infrastructure and included three years of growth resulting in an increased replacement value.

The key messages from the 2024-24 Parks & Environment Asset Management Plan are summarised below:

Asset Data & Condition Analysis

- The data utilised to develop the PEAMP is considered to be approximately 95% accurate and of medium confidence. The recent condition ratings were established from a network assessment conducted by Talis in August, 2019.

- The PEAMP infrastructure assets are in excellent condition with 75% of the hard infrastructure park assets in condition 1 and 2, 20% in condition 3 and 5% in condition 4 and 5.

Level of Service and Risk Management

Level of Service is a measurable target which determines the type and extent of services delivered to the Community. The following findings have been drawn from the CATALYSE Pty Community Scorecard Report April 2020.

- Community satisfaction for the City’s playgrounds, parks and reserves is very positive, with 94% of those surveyed rating performance as either excellent or good.

- Community satisfaction with the City’s conservation and environmental management is also high with 90% rating performance as either excellent or good.

- Existing controls and expenditure to mitigate risk are considered adequate, thus reducing the impact on service delivery.

See (Section 3) for further information

Future Growth and Demand Management

Future growth projections are supported by the City’s Community Strategic Plan population and demographic research, whilst Demand for new services will be administered through upgrading existing and providing new assets.

- An average growth of 19 Ha of parks and recreation land per year over the next 5 years. This represents a 1% growth per annum.

See (Section 4) for further information

Lifecycle Cost Management

The lifecycle management section details how the City plans to manage and operate both current and future assets to the agreed levels of service whilst optimising life cycle costs.

- Maintenance expenditure levels are considered to be adequate to meet current service levels.

- Planned maintenance work represents 86% of total Parks Service Unit maintenance expenditure and 73% of total Environment Service Unit maintenance expenditure.

- By 2029-30 required expenditure for Operations and Maintenance is expected to be around $27.7m per year for Parks and $8m for Environment. Including a 2% CPI increase per year forward.

See (Section 5) for further information

Financial Analysis

Renewal Forecasts

The City has developed a 10 year renewal plan which will drive the budget planning process and form the basis to the City’s long term financial planning.

- There are currently $1.08m of irrigation assets that are considered to be beyond their useful life.

- Overall the majority of the City’s Parks and Environment assets are in a good condition with 75.9% of the assets currently rating as either a 1 or a 2. (Excellent or Good)

- 10 years projections indicate a potential cumulative funding gap estimated at $0.3m for all parks and environment hard infrastructure if the allocated renewal budget figures from the LTFP are adhered to.

See (Section 6) for further information

Sustainability of Service Delivery

The City will comply and report its Parks & Environment hard infrastructure assets performance in relation to the Department of Local Government’s Asset Management Guidelines and Framework.

Based on actual expenditure in 2019-20, the following table indicates the City’s performance in managing our parks and environment assets.

| Asset group | Consumption ratio 2019-20 | Sustainability ratio 2020-21 | Renewal funding ratio 10 year |
| --- | --- | --- | --- |
| Infrastructure | 68.3% | 1% | 100% |
| Irrigation | 63.9% | 37% | 81% |
| Playgrounds | 67.7% | 16% | 97% |
| Total All Assets | 67.4% | 13% | 93% |
| Dept. of LG framework Standards | Improving | Not met | Improving |

Sustainability ratios have been forecast for the next 10 years to reflect the improvements the City is making following completion of the LTFP. The sustainability ratio for 2029/30 for all assets is predicted to be 33%.

See (Section 6.3) for further information

Plan Improvement Strategy and Monitoring

Most of the strategic improvements identified in the last PEAMP are now complete.

Further improvements that will continue to develop future revisions of the plan have been identified.

- The next audit and asset data pick up to assess physical condition of all parks assets to be completed in 2022/23.

- Monitoring performance measures against levels of service by continually reviewing historic service level data prior to financial year commence.

See (Section 8) for further information

# 2.	Introduction

## 2.1	Background

This asset management plan has been developed to assist the Parks and Environment Business Unit to outline the management of assets, compliance with regulatory requirements and to highlight the funding required to provide the appropriate Levels of Service.

The assets covered by this plan are summarised in Table 2.1.1. Figures as at November 2020, have been extracted from Council’s Technology One Enterprise Asset Management System (EAM).

Table 2.1.1 	Depreciable Assets (parks and environment hard infrastructure)

| Asset Class | Asset Group | Asset type | Number of | Replacement Value |
| --- | --- | --- | --- | --- |
| INFRASTRUCTURE | Amenities | BBQ | 92 | $7,886,395 |
|  |  | Bicycle racks | 114 |  |
|  |  | Drink Fountains | 146 |  |
|  |  | Picnic settings/shelters | 321 |  |
|  |  | Seats | 1,073 |  |
|  |  | Showers | 14 |  |
|  |  | Taps | 20 |  |
|  | Bins | Dog Pouch Stations | 269 | $731,177 |
|  |  | Rubbish Bins | 663 |  |
|  | Fence | Baseball fence (m) | 61 | $35,003,282 |
|  |  | Bollards (no.) | 17,958 |  |
|  |  | Chain Wire / Wire (m) | 52,852 |  |
|  |  | Chicanes (no.) | 46 |  |
|  |  | Colorbond (m) | 713 |  |
|  |  | Conservation (m) | 3,874 |  |
|  |  | Feral / rabbit proof (m) | 7,620 |  |
|  |  | Gates (all types) | 412 |  |
|  |  | Handrails / railings (m) | 12,225 |  |
|  |  | Netting – various (m) | 701 |  |
|  |  | Retaining walls (m) | 28,980 |  |
|  |  | Ringlock (m) | 22,178 |  |
|  |  | Walls (m) | 10,654 |  |
|  | Lighting | Bollards | 64 | $6,477,525 |
|  |  | CCTV Cameras | 6 |  |
|  |  | Directional | 633 |  |
|  |  | Feature | 409 |  |
|  |  | Floodlights | 151 |  |
|  |  | Midge Lights | 11 |  |
|  |  | Solar lights | 65 |  |
|  | Minor Structure | Amphitheatre | 2 | $14,911,466 |
|  |  | BMX Facility | 7 |  |
|  |  | Basketball / Netball / Tennis court | 78 |  |
|  |  | Bowls (synthetic) | 6 |  |
|  |  | Cricket pitch | 58 |  |
|  |  | Discus / shotput throwing circle | 4 |  |
|  |  | Gazebo | 105 |  |
|  |  | Jetties | 4 |  |
|  |  | Long jump pit | 12 |  |
|  |  | Shelter | 186 |  |
|  |  | Skate Facility | 10 |  |
|  |  | Tree Protector | 137 |  |
|  |  | Undersurface | 836 |  |
|  | Signs | Dog Pouch Stations | 459 | $1,553,044 |
|  |  | Entry / Locality Statements | 9 |  |
|  |  | Environmental trail markers | 23 |  |
|  |  | Environmental boundary signs | 20 |  |
|  |  | Information signs | 146 |  |
|  |  | Interpretive / directional signs | 109 |  |
|  |  | Plaques | 32 |  |
|  |  | Park name signs | 59 |  |
|  |  | Parking signs | 70 |  |
|  |  | Warning signs | 318 |  |
|  | Sporting Equipment | Basketball / netball rings | 114 | $2,398,895 |
|  |  | Cricket (practice nets) | 22 |  |
|  |  | Disc Golf holes | 9 |  |
|  |  | Exercise equipment | 93 |  |
|  |  | Goals | 98 |  |
| IRRIGATION |  | Aerators | 10 | $18,144,416 |
|  |  | Bores & pumps | 237 |  |
|  |  | Discharge Assembly | 222 |  |
|  |  | Electrical Cabinets | 269 |  |
|  |  | Infield pipes | 2,535,400 |  |
|  |  | Iron filtration unit | 9 |  |
|  |  | Sprinkers (gear drive) | 22,431 |  |
|  |  | Sprinklers (pop ups) | 11,490 |  |
|  |  | Valves | 2,497 |  |
|  |  | Wet Well | 1 |  |
| PLAYGROUNDS |  | Playgrounds | 394 | $22,082,656 |
|  |  | Play Equipment | 382 |  |
|  |  | Water Play Equipment | 2 |  |
|  |  | Shade structures | 128 |  |
| TOTAL Depreciable Assets |  |  |  | $109,188,856 |

Table 2.1.2 	Non-Depreciable Asset (parks and environment soft infrastructure)

| Park Classification | Total area (Ha) |
| --- | --- |
| Bushland | 1071.19 |
| Community | 49.97 |
| Major Passive | 20.82 |
| Minor Passive | 288.48 |
| Playing Fields | 159.92 |
| SAR* Parks | 8.37 |
| Streetscape | 271.62 |
| TOTAL | 1870.37 |

*SAR (Special Area Rate) – primarily applied for the increased service level of parks and streetscapes in the Port Coogee precinct.

The above table was correct for the total areas as at February 2021.

The AMP is to be read in conjunction with the following associated planning documents:

City of Cockburn Strategic Community Plan 2020 – 2030

City of Cockburn Corporate Business Plan 2016/17 – 2019/20

City of Cockburn Annual Business Plan 2019 – 2020

City of Cockburn Long Term Financial Management Plan 2019/20 – 2032/33

Key stakeholders in the preparation and implementation of this asset management plan are shown in Table 2.1.2

Table 2.1.2	Key Stakeholders in the AM Plan

| ENTITY: | NATURE OF INVOLEMENT |
| --- | --- |
| INTERNAL STAKEHOLDERS: |  |
| The Elected Council | Community representation |
| Chief Executive Officer (CEO) | Asset management direction and leadership |
| Executive Committee (ExCo) | Executive management endorsement, sign off and executive ownership |
| Chief of Operations | Review and strategic management sign off |
| Head of Operations | Review and line management sign off and implementation of the AMP maintenance actions |
| Property and Assets Services | Asset Management Plan development, review and continuous improvement |
| EXTERNAL STAKEHOLDERS: |  |
| Insurers | Assist to manage financial risk of the City |
| City of Cockburn community Business Perceptions Survey Community | Service users |
| City of Cockburn business Police, Fire and Emergency Services | Service users |

## 2.2	Goals and Objectives of Asset Management

The City of Cockburn exists to deliver services to its community.  Some of these services are provided by infrastructure assets.  The City has acquired infrastructure assets by ‘purchase’, by contract, construction by council staff and by donation or gifting of assets constructed by developers and others to meet increased levels of service.

The City of Cockburn’s goal in managing infrastructure assets is to meet the required level of service in the most cost effective manner for present and future consumers.  The key elements of infrastructure asset management are:

- Taking a life cycle approach,

- Developing cost-effective management strategies for the long term,

- Providing a defined level of service and monitoring performance,

- Understanding and meeting the demands of growth through management and infrastructure investment,

- Managing risks associated with asset failures,

- Sustainable use of physical resources,

- Continuous improvement in asset management practices.

This asset management plan is prepared under the direction of Council’s vision, mission, goals and objectives.

The City of Cockburn’s vision is:

Cockburn, the best place to be

The City of Cockburn’s purpose is:

Support our communities to thrive by providing inclusive and sustainable services which reflect their aspirations

The 5 key outcomes as detailed in the Strategic Community Plan (SCP) 2020-2030 are:

- Local Economy,

- Environmental Responsibility

- Community, Lifestyle & Security

- City Growth and Moving Around

- Listening and Leading

The relevant goals and objectives as outlined in the Strategic Community Plan and how these are addressed in this asset management plan are detailed in Table 2.2.1.

Table 2.2.1 – Council Goals and how these are addressed in this Plan

| Strategic Outcome | Strategic Objective | How Outcomes and Objectives are addressed in this plan |
| --- | --- | --- |
| Environmental Responsibility A leader in environmental management that enhances and sustainably manages our local natural areas and resources | 1. Sustainable resource management including waste, water and energy 2. Protection and enhancement of our natural areas, bushland, parks and open spaces | Future Growth and Demand: Section 4 Levels of Service: Section 3 |
| Community, Lifestyle and Security A vibrant, healthy, safe, inclusive and connected community | 1. Accessible and inclusive community, recreation and cultural services and facilities that enrich our community | Levels of Service: Section 3 |
| Listening and Leading A community focused, sustainable, accountable and progressive organisation | 1. Best practice Governance, partnerships and value for money 2. High quality and effective community engagement and customer service experiences | Financial Analysis: Section 6 Levels of Service: Section 3 |

## 2.3 Plan Framework

Key elements of the plan are

- Levels of service – specifies the services and levels of service to be provided by council.

- Future demand – how this will impact on future service delivery and how this is to be met.

- Life cycle management – how Council will manage its existing and future assets to provide the required services

- Financial summary – what funds are required to provide the required services.

- Asset management practices

- Asset management monitoring and improvement plan -  how the plan will be monitored to ensure it is meeting Council’s objectives

## 2.4 Asset Management Maturity

The 2020-2024 PEAMP has been developed in accordance with the International Infrastructure Management Manual (IIMM) and complies with Department of Local Government & Communities Asset Management Framework.

As part of the City’s Strategic Asset Management Planning Framework (SAMPF), the PEAMP will formalise the City’s future forecasting for Parks and Environment Infrastructure, enabling the organisation to determine future budgeting requirements, sustain the current and future asset base, whilst ensuring that optimisation of activities and programs facilitate for the capture and reporting of adopted service levels.

The plan has reached an ‘intermediate’ level of maturity and provides Executive level monitoring and reporting of key improvement areas from the Improvement Strategy.

With the continued implementation of the Strategic Asset Management Planning Framework, the City will commence measuring service levels for planned and reactive maintenance to determine operational performance and asset utilisation.

The City strives to improve its strategic and operational asset management practices and to continue its journey towards advanced asset management. The Department of Local Government, Sport and Cultural Industries (DLGSC) has developed the Western Australia Local Government Integrated Planning and Reporting Framework. The future direction and need for advanced level practices are continually assessed in accordance with this and the City’s Asset Management Policy. The Integrated Planning and Reporting Framework is shown Figure 2.4.1.

Figure 2.4.1 The City’s Corporate Planning Framework

The PEAMP forms part of the City’s Assets Informing Strategies, which consists of the following strategy and asset management plans:

Asset Management Strategy – 2017 – 2024

Buildings AMP – 2020 – 2024

Cockburn Aquatics and Recreation Centre (ARC) AMP – 2020 – 2024

Drainage AMP – 2020 – 2024

Footpath AMP – 2020 – 2024

Fleet and Plant AMP – 2020 -2024

Marina and Coastal Infrastructure AMP – 2020 – 2024

Road Infrastructure AMP – 2020 – 2024

## 2.5 Asset Management Plan – Data confidence assessment

Each of the five sections within the PEAMP were reviewed to determine Stakeholder confidence as to the accuracy and maturity of the City’s asset data and services.

| AMP | Contents | Data Accuracy |
| --- | --- | --- |
| Section 2 | Strategic goals & objectives | A |
| Section 3 | Levels of Service Risk Management | B |
| Section 4 | Growth, Demand, New Assets | B |
| Section 5 | Asset data; Age, Condition Operating & Maintenance Expenditure, Renewal Expenditure | B |
| Section 6 | Financial statements; Renewals Gap, Ratios | A |

Ratings are based on the following criteria / inputs.

| Confidence Grade | Description |
| --- | --- |
| A Highly reliable | Data based on sound records, procedures, investigations and analysis, documented properly and agreed as the best method of assessment. Dataset is complete and estimated to be accurate ± 2% |
| B Reliable | Data based on sound records, procedures, investigations and analysis, documented properly but has minor shortcomings, for example some of the data is old, some documentation is missing and/or reliance is placed on unconfirmed reports or some extrapolation. Dataset is complete and estimated to be accurate ± 10% |
| C Uncertain | Data based on sound records, procedures, investigations and analysis which is incomplete or unsupported, or extrapolated from a limited sample for which grade A or B data are available. Dataset is substantially complete but up to 50% is extrapolated data and accuracy estimated ± 25% |
| D Very Uncertain | Data is based on unconfirmed verbal reports and/or cursory inspections and analysis. Dataset may not be fully complete and most data is estimated or extrapolated. Accuracy ± 40% |
| E Unknown | None or very little data held. |

# 3. Levels of Service

## 3.1 Customer Research and Expectations

To support the management of Parks and Environment assets the City has developed industry best practice asset management and customer focused levels of service (LOS) for infrastructure assets and associated services. These LOS’s provide the City with a mechanism to deliver operational activities that endeavour to meet community expectations in the most cost effective manner possible.

The City administers community and technical service levels to ensure that quality service provision is provided in accordance with the City’s customer Service Charter and Community Engagement Framework, whilst technical services are sustainable, and adhere to all relevant compliance and safety industry standards.

The PEAMP community and technical levels of service are defined to an asset group level and enable the City to monitor and report operational performance against adopted community and technical targets.

PEAMP service levels are derived from the City’s Enterprise Asset Management System (EAM). The Implementation of EAM has improved reporting of operational and maintenance budget expenditure providing increased confidence in projecting future budget needs.

The City of Cockburn administered the CATALYSE Pty Ltd Business and Community Perceptions Survey’s to evaluate and monitor performance across a range of services.

697 Residents and 138 local businesses participated in the studies. The surveys provide Council with valid performance measures that can be benchmarked and consistently monitored over time.

The most recent Community satisfaction survey was undertaken in April 2020 and the Business alternative in October 2019, surveys are referred to below as 2019 as they’re financial year 2019/20. Performance comparison of satisfaction levels over the past five years are compared using a traffic light system to measure increasing or decreasing satisfaction.

Key to status reported below:

Drop in customer satisfaction of 3% or more

Change in customer satisfaction of 2% or less

Increase in customer satisfaction of 3% or more

Historical tracking of customer satisfaction surveys

Table 3.1.1 Community, April 2020

Satisfaction remains moderate through the years for Playgrounds and Parks with 94% of residents satisfied, although this is a priority area for continued improvement.

| Performance Measure | Satisfaction Level (delighted & satisfied) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2014-15 | 2015-16 | 2016-17 | 2017-18 | 2019-20 | Status from last year |
| Overall satisfaction with City of Cockburn (as a place to live) | 95 | 93 | 99 | 97 | 98 |  |
| Playgrounds and parks (inc. sporting ovals up to 2013) | 91 | 94 | 95 | 95 | 94 |  |
| Streetscapes | 78 | 82 | 82 | 79 | 73 |  |
| Conservation and Environmental management | 78 | 81 | 91 | 90 | 90 |  |

Table 3.1.2 Business, October 2019

| Performance Measure | Satisfaction Level (delighted & satisfied) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2014-15 | 2015-16 | 2016-17 | 2017-18 | 2019-20 | Status from last year |
| Overall satisfaction with City of Cockburn (as a business location) | 85 | 89 | 95 | 93 | 91 |  |
| Streetscapes | 57 | 69 | 84 | 85 | NA |  |
| Conservation and Environmental management | 78 | 70 | 89 | NA | 86 |  |

The City of Cockburn uses this information to continue developing the Community Strategic Plan and determine the allocation of resources to meet the community’s needs.

## 3.2 Current Levels of Service

The City of Cockburn has defined service levels in two terms:

- Community Levels of Service relate to how the community receives the service in terms of safety, quality, quantity, reliability, responsiveness, cost efficiency and legislative compliance.

- Supporting the community service levels are operational or technical measures of performance developed to ensure that at least the minimum community levels of service are met. Technical Levels of Service relate to how the City provides the service using technical terms

Data for the SAR parks is managed outside of this AMP as they are currently maintained by an external contractor.

Table 3.1.3 	Current Service Levels – Irrigation Infrastructure

| Level of Service | 2017-18 | 2018-19 | 2019-20 |
| --- | --- | --- | --- |
| COMMUNITY LEVELS OF SERVICE |  |  |  |
| o Water quality testing undertaken every year to ensure that water quality is suitable (site specific) | Met | Met | Met |
| o Irrigation infrastructure is reliable and efficient and does not conflict with the public. | 347 | 422 | 370 |
| Number of customer enquiries relating to irrigation. |  |  |  |
| o No trip hazards or holes present that could cause injury resulting in public liability claims | No claims | No claims | No claims |
| TECHNICAL LEVELS OF SERVICE |  |  |  |
| o Irrigation maintenance programs are undertaken to: |  |  |  |
| 1. Playing fields 52 services p/a | Not met | Not met | Not met |
| 2. SAR parks 39 services p/a | Met | Met | Met |
| 3. Community 26 services p/a | Not met | Not met | Not met |
| 4. Major passive 26 services p/a | Not met | Not met | Not met |
| 5. Minor Passive 26 services p/a | Not met | Not met | Not met |
| 6. Streetscapes 26 services p/a | Not met | Not met | Not met |
| See graph and tables below for the number of estimated hours and scheduled visits against actual hours maintenance and visits completed by park hierarchy for year 18/19 and 19/20 |  |  |  |
| o Compliance with the Water Efficiency Action Plan to continually reduce groundwater abstraction as per 2017/18 levels | 2,217,640 kl | 2,495,835 kl | 2,610,238 kl |
| o To reduce power consumption all sites are to be on R1 Power tariffs | 90% | 90% | 90% |

Table 3.2.4 	Current Service Levels – Playgrounds

| Level of Service | 2017/18 | 2018-19 | 2019-20 |
| --- | --- | --- | --- |
| COMMUNITY LEVELS OF SERVICE |  |  |  |
| o Playground equipment is well maintained and valued by the community. | 248 | 306 | 275 |
| Number of customer enquiries relating to Playgrounds (see below for breakdown of customer request types) |  |  |  |
| o Playground equipment meets user requirements. 100% of requests completed as per agreed service level. | Met | Met | Met |
| o Playgrounds are safe, clean and free from hazards that could result in public liability claims | 0 Claims | 1 Claim | 1 Claim |
| TECHNICAL LEVELS OF SERVICE |  |  |  |
| o Independent audits are undertaken on all playground equipment: |  |  |  |
| 1. Major audit 1 per annum | Met | Met | Met |
| 2. Minor audit 2 per annum | Not met | Not met | Not met |
| o Completion of annual playground replacement program to ensure that all equipment is in a good working order | Met | Met | Met |
| o Where appropriate facilities are accessible to all users and meet with DAIP requirements. | Met | Met | Met |
| o All playgrounds and equipment are safe and free from hazards by ensuring that all reactive maintenance is completed. | Met | Met | Met |

The following table shows the breakdown in the type of customer requests received regarding playgrounds

| CRM Sub Type | No of |  |  | CRM sub type | No of |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2017-18 | 2018-19 | 2019-20 |  | 2017-18 | 2018-19 | 2019-20 |
| Equipment | 90 | 112 | 91 | None | 3 | 7 | 4 |
| Exercise equipment | 7 | 8 | 7 | Other park | 68 | 92 | 96 |
| Litter | 14 | 25 | 27 | Replacement | 28 | 28 | 12 |
| New | 14 | 6 | 16 | Shade sail | 24 | 28 | 22 |

Table 3.2.5 	Current Service Levels – Park Furniture

| Level of Service | 2017-18 | 2018-19 | 2019-20 |
| --- | --- | --- | --- |
| COMMUNITY LEVELS OF SERVICE |  |  |  |
| o Park furniture is clean, accessible, comfortable and located in appropriate areas and environments | 466 | 556 | 584 |
| Number of customer enquiries relating to furniture |  |  |  |
| o All park furniture is safe and free from hazards that may result in public liability claims | No Claims | No Claims | 1 Claim |
| TECHNICAL LEVELS OF SERVICE |  |  |  |
| o Park furniture is maintained in a clean, good condition and free from graffiti by ensuring that annual audit, reactive maintenance and replacement program are completed. | Met | Met | Met |
| o 10% of all new park furniture installed is renewable | Met | Met | Met |
| o New park furniture to accommodate universal access where required and complies with DIAP. | Met | Met | Met |

Table 3.2.6 	Current Service Levels – Playing Fields and Open Space

| Level of Service | 2017-18 | 2018-19 | 2019-20 |
| --- | --- | --- | --- |
| COMMUNITY LEVELS OF SERVICE |  |  |  |
| o That all playing fields are fit for their purpose and meet user requirements. | 50 | 56 | 63 |
| Number of customer enquiries relating to turf on Playing fields |  |  |  |
| o That all playing fields are available for training, competition and events and sporting club requests are completed | Met | Met | Met |
| o Playing fields are even and safe and free from hazards that could result in public liability claims | No Claims | No Claims | 1 Claim |
| TECHNICAL LEVELS OF SERVICE |  |  |  |
| o Maintenance schedules are undertaken to: |  |  |  |
| 1. Playing fields 52 services p/a | Met | Met | Met |
| 2. SAR parks 39 services p/a | Met | Met | Met |
| 3. Community Facilities 26 services p/a | Met | Met | Met |
| 4. Major passive 26 services p/a | Met | Met | Met |
| 5. Minor Passive 26 services p/a | Met | Met | Met |
| 6. Streetscapes 26 services p/a | Met | Met | Met |
| o Scheduled fertiliser and herbicide programs are undertaken to: |  |  |  |
| 1. Playing fields 4 applications p/a from 17/18 2 applications p/a | Met | Met | Met |
| 2. Major passive 2 applications p/a from 17/18 1 applications p/a | Met | Met | Met |
| o Machine hours are monitored to provide an efficient and effective use of resources | Met | Met | Met |

Table 3.2.7 	Current Service Levels – Road Reserves and Streetscapes

| Level of Service | 2017-18 | 2018-19 | 2019-20 |
| --- | --- | --- | --- |
| COMMUNITY LEVELS OF SERVICE |  |  |  |
| o All areas are free from rubbish. | 60 | 71 | 65 |
| Number of Customer enquiries relating to litter. |  |  |  |
| o All areas are neat and well maintained. | 51 | 53 | 63 |
| Number of customer enquiries relating to Streetscapes |  |  |  |
| o All areas are free from hazards that could result in public liability claims | 2 | 3 | 2 |
| TECHNICAL LEVELS OF SERVICE |  |  |  |
| o Maintenance schedules undertaken to: |  |  |  |
| 1. Priority 1 roads 6 services p/a | 4 | 4 | 4 |
| 2. Priority 2 roads 4 services p/a | 2 | 2 | 2 |
| 3. Rural roads 2 services p/a | Met | Met | Met |
| 4. Industrial areas 2 services p/a | Met | Met | Met |
| 5. Unkempt verge maintenance 4 services p/a on request | Met | Met | Met |
| o Machine hours are monitored to ensure efficient and effective use of resources | Met | Met | Met |

The following tables show the number of visits scheduled by park hierarchy and the number of visits actually completed. This data was taken from the City’s Asset Management System for the year 2019-20. If a work order has no hours recorded against it then it has been assumed that no visit was completed.

The graph below show the total number of maintenance hours scheduled (estimated) in the work orders against the number of actual hours entered per work order for the year 2019-20 in comparison to 2018-19.

For a breakdown of the maintenance expenditure refer to Section 5.2

Irrigation

Number of visits completed

|  | No of scheduled visits | No of visits completed | % Completed 18/19 | % Completed 19/20 |  |
| --- | --- | --- | --- | --- | --- |
| Community | 522 | 332 | 60.0% |  | 63.6% |
| Major Passive | 183 | 137 | 77.5% |  | 74.9% |
| Minor Passive | 3557 | 2320 | 69.3% |  | 65.2% |
| Playing Fields | 1245 | 982 | 75.5% |  | 78.9% |
| Streetscapes | 1001 | 495 | 53.4% |  | 49.5% |
| TOTAL | 6508 | 4266 | 67.5% |  | 65.6% |

Irrigation – Estimated hours against actual hours

Mowing

Number of visits completed

|  | No of scheduled visits | No of visits completed | % Completed 18/19 | % Completed 19/20 |
| --- | --- | --- | --- | --- |
| Community | 435 | 299 | 67.7% | 68.7% |
| Major Passive | 249 | 151 | 67.2% | 60.6% |
| Minor Passive | 4237 | 3066 | 72.5% | 72.4% |
| Playing Fields | 625 | 494 | 79.6% | 79.0% |
| Streetscapes | 1075 | 650 | 61.5% | 60.5% |
| TOTAL | 6621 | 4660 | 70.9% | 70.4% |

Mowing – Estimated hours against actual hours

Tractor Mowing

Number of visits completed

|  | No of scheduled visits | No of visits completed | % Completed 18/19 | % Completed 19/20 |
| --- | --- | --- | --- | --- |
| Community |  |  |  |  |
| Major Passive | 79 | 52 | 70.5% | 65.8% |
| Minor Passive | 410 | 325 | 85.5% | 79.3% |
| Playing Fields | 1232 | 1027 | 85.7% | 83.4% |
| Streetscapes |  |  |  |  |
| TOTAL | 1721 | 1404 | 85.0% | 81.6% |

Tractor mowing - Estimated hours against actual hours

Landscape Maintenance

Number of visits completed

|  | No of scheduled visits | No of visits completed | % Completed 18/19 | % Completed 19/20 |
| --- | --- | --- | --- | --- |
| Community | 522 | 332 | 66.3% | 63.6% |
| Major Passive | 183 | 137 | 75.5% | 74.9% |
| Minor Passive | 3557 | 2320 | 60.9% | 65.2% |
| Playing Fields | 1245 | 982 | 66.9% | 78.9% |
| Streetscapes | 1001 | 495 | 44.0% | 49.5% |
| TOTAL | 6508 | 4266 | 52.6% | 65.6% |

Landscape Maintenance – Estimated hours against actual hours

The City continues to relocate, remove or replace trees historically installed in inappropriate locations within the streetscapes. New Street tree plantings are now more carefully considered for long term establishment as per current service levels below.

Table 3.3.6	Current Service Levels – Trees

| Level of Service | 2017-18 | 2018-19 | 2019-20 |
| --- | --- | --- | --- |
| COMMUNITY LEVELS OF SERVICE |  |  |  |
| o Trees are in an appropriate location, healthy and disease free | 2242 | 2635 | 3264 |
| Number of customer enquiries relating to trees |  |  |  |
| o Branches are kept clear of power lines by ensuring that all Western Power notifications are completed on time | Met | Met | Met |
| o Trees are managed through the street tree management strategy and plan | Met | Met | Met |
| TECHNICAL LEVELS OF SERVICE |  |  |  |
| o 100% of customer requests are completed | Met | Met | Met |
| o Trees are maintained and protected as per council policy PSEW15 and the annual tree pruning program | Met | Met | Met |

The following graph shows the breakdown in the type of CRM requests received for the past two years with regards to trees.

Current Service Levels – Environment

| Level of Service | 2017-18 | 2018-19 | 2019-20 |
| --- | --- | --- | --- |
| COMMUNITY LEVELS OF SERVICE |  |  |  |
| o Bushland and surrounds are kept healthy and weed free. Number of customer enquiries relating to bushland reserves. | 56 | 93 | 173 |
| o Midge levels are controlled. Number of customer enquiries relating to midges | 17 | 10 | 4 |
| o Natural areas provide habitat for local fauna and flora. Number of customer enquiries relating to fauna | 9 | 26 | 25 |
| TECHNICAL LEVELS OF SERVICE |  |  |  |
| o 70% of bushland vegetation is in a good or better condition | 69% | 73% | 75% |
| o 70% or more of mapped^ actively managed reserves have no high priority weed species* mapped as 61% or greater density | Achieved 74% | Achieved 90% | Achieved 94% |
| o All firebreaks are maintained and in a good condition | Achieved | Achieved | Achieved |

*based on City of Cockburn high priority weed species list.

^Weed mapping occurs annually by way of mapping a sub-set of the reserves on a 4 year rotational basis, thus each year reserves differ from the previous year, but a comparable dataset is produced over a four year period.

Number of Work Orders (WO) generated from Customer Service enquiries (ENQ) for years 2018-19 and 2019-20

The following table shows the number of customer enquiries and/or requests received that resulted in maintenance being required via reactive work orders.

| ASSET CLASSIFICATION | 2018-19 |  |  | 2019-20 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ENQ | WO | % | ENQ | WO | % |
| Furniture | 556 | 8 | 1.4% | 584 | 6 | 1.0% |
| Horticulture | 99 | 6 | 6.1% | 114 | 8 | 7.0% |
| Irrigation | 422 | 4 | 0.9% | 370 | 0 | 0% |
| Lighting | 65 | 0 | 0% | 60 | 0 | 0% |
| Minor structures | 78 | 2 | 2.6% | 113 | 0 | 0% |
| Playgrounds | 306 | 20 | 6.5% | 275 | 25 | 9.1% |
| Sporting structure | 47 | 0 | 0% | 51 | 0 | 0% |
| Trees | 2635 | 63 | 2.4% | 3264 | 29 | 0.9% |
| Turf | 916 | 21 | 2.3% | 1114 | 13 | 1.2% |
| TOTAL | 5124 | 124 | 2.42% | 5945 | 81 | 1.36% |

The percentage column shows the number of work orders as a percentage of the customer enquiries received. The total number of enquiries has increased however there is a decrease in the number of reactive work orders generated for the same period.

## 3.3 Enterprise Risk Management

In 2015 the City implemented a Risk Management & Safety System (RMSS) in which all operational and strategic risks are captured, rated and receive ongoing monitoring based on their level of risk.

Additionally, in 2017 the Risk Management Framework was adopted with the aim of supporting an integrated and effective organisational wide approach to risk management.

The implementation of the Framework will:

1. Ensure a consistent approach to the risk management process across Council

2. Establish a structured process for undertaking the risk management process to identify, assess and control/treat risk

3. Encourage the integration of risk management into the strategic and operational process across all Business Units

There are currently no Extreme or High Risks identified that are associated to the Parks and Environment Service Unit.

Each of the risks are reviewed with current and proposed control measures being assessed yearly to ensure industry standards and potential advancements are considered and are incorporated as required.

The City uses a matrix based approach when addressing risk level, treatment and responsibility as detailed in Table 3.3.1.

Table 3.3.1	Risk Treatment Matrix

| Risk Level | Code | Criteria | Treatment | Responsibility |
| --- | --- | --- | --- | --- |
| LOW | L | Risk acceptable with adequate controls, managed by routine procedures. Subject to annual monitoring or continuous review throughout project lifecycle. | Management through routine operations/project, Risk Registers to be updated. | Service Unit Manager/Project Manager |
| MODERATE | M | Risk acceptable with adequate controls, managed by specific procedures. Subject to semi- annual monitoring or continuous review throughout project lifecycle. | Communication and awareness of increasing risk provided to SM, Risk Registers to be updated. | Senior Manager/Project Manager |
| SUBSTANTIAL | S | Accepted with detailed review and assessment. Action Plan prepared and continuous review. | Assess impact of competing Service Unit/Business Unit Projects. Potential redirect of Service Unit/Business Unit resources. Risk registers to be updated. | Director/Steering Committee |
| HIGH | H | Risk acceptable with effective controls, managed by senior management/executive. Subject to quarterly monitoring or continuous review throughout project lifecycle. | Escalate to CEO, report prepared for Audit & Strategic Finance Committee. Quarterly monitoring and review required. Risk Registers to be updated. | Executive/ Steering Committee/Project Sponsor |
| EXTREME | E | Risk only acceptable with effective controls and all treatment plans to be explored and implemented where possible, managed by highest level of authority and subject to continuous monitoring. | Escalate to CEO, report prepared for Audit & Strategic Finance Committee. Monthly monitoring and review required. Risk Registers to be updated. | CEO/Council/Project Sponsor |

## 3.4 Legislative Requirements

The City has to meet many legislative requirements including Australian and State legislation and State regulations.  These are shown in Appendix A.

## 3.5 Asset Capacity and Performance

The City of Cockburn services are generally provided to meet design and performance standards where these are available.

Locations where deficiencies in service performance are known have been identified by the Parks & Environment Service unit and are detailed in the following table.

Table 3.5	Known Service Performance Deficiencies

| Location | Service Deficiency |
| --- | --- |
| Irrigation | Irrigation infrastructure is ageing at various rates which have the potential to fail prior to identify lifecycle, impacting on delivery of service and reserve management. An increase in irrigation infrastructure through new subdivisions and upgrades to the City’s road network has led to a reduced maintenance service level. More recently the City has been performing reactive maintenance only with regards to asset replacement. |
| Groundwater Management | There has been and will continue to be an increase in the regulatory framework on the management of the abstracted groundwater which is impacting on the existing resources. New resources are required to meet the changes and ensure no breaches of the ground water license conditions. Working collaboratively with contracted company to assist with groundwater monitoring and reporting. Require integrated software in conjunction with central irrigation software to assist with process. |
| Sport Fields / Passive Reserves | Inconsistent turf quality across a majority of reserves is due to inefficient irrigation systems, reduced water allocations, high usage on sporting ovals through an increase in sporting clubs and limited window to conduct turf renovation between summer and winter sports. Current service levels are inconsistent with budget and resource allocations. Poor quality turf is also due to poor soil health, provision may need to be made to improve the structure using compost. With compost added soils will hold water longer making more sustainable use of groundwater. Changed irrigation installation method which is now cheaper and more efficient to replace. Contracted out mowing of some premier sports fields to produce quality turf surface, there are still improvements to be made. |
| Playgrounds | The continued increase in playgrounds through new subdivisions and the increase in adventure / nature playgrounds further exacerbate the variance between levels of service and budget allocations. |
| Street Trees | Current service is based on reactive street tree pruning by request, development of a Tree management plan will outline resourcing and funding. Annual audit on specific trees within Parks and Street Tree Audit every 3 years until development of a Tree Management Plan. |
| Verge Maintenance | Current service is based on reactive mowing of verges by residential requests an increase service delivery will mitigate complaints and improve aesthetic appearance for the wider community. |
| Streetscapes | An increase in streetscapes via developer contributions are not identified accurately thereby reducing service level across all streetscapes. Landscaped streetscapes are increasing and are difficult for residents to maintain as they are disconnected by laneways and rear property access. |
| Natural Areas | Maintenance service levels based on current funding are inadequate to complete full range of work actions. |
| Signage | Lack of appropriate and adequate signage prevents the delivery of valid information thereby reducing the experience for the community. |
| Midge Lights | An increase in service level required to ensure functionality of infrastructure and community expectations. |
| Boardwalks / Lookouts | An increase in service level required to ensure functionality of infrastructure and community expectations. |
| Sports Oval Lighting | There has been an increased demand to utilizing sporting reserve at night through higher participation rates at sporting clubs requiring an increase to service and maintain the luminaire lighting levels. Meeting this new demand requires additional funding and resources along with a plan to replace the existing lamps with LED. |
| Verge/Crossover Compliance | There has been an increasing trend towards property owners installing non-compliant verges and crossovers which impacts the current resources within the business unit. Some are dangerous and risk must be considered, this continues to be a deficiency. |

The above service deficiencies were identified from the previous and current Parks service unit annual budget and the Environment service unit budgets along with an update provided by internal staff.

# 4. Future Growth and Demand

## 4.1	Growth Forecast

Cockburn is one of the major Coastal Cities found in the state of Western Australia, totalling 170 square kilometres. This coastal City is renowned for its historical and tourism features along with agriculture and ship building industries. One of the City’s most unique features is a chain of five lakes running north to south through the heart of the City. Sixteen Aboriginal campsites have been identified throughout Cockburn, most of them on the fringes of two of the larger lakes, North Lake and Bibra Lake.

The City of Cockburn’s 2020 forecasted population and dwelling is 120,417 and 46,800 dwellings respectively. The population is forecast to reach 169,700 by 2041, an increase of 40.92%.

Growth factor trends and the impacts these have on service delivery across the City are summarised in Table 4.1.

Table 4.1 Demand Factors, Projections and Impact on Services

| Demand factor | Present position | Projection | Impact on services |
| --- | --- | --- | --- |
| Population | 120,417 as at year 2020 | Change between 2019 and 2041 is projected to be 52,176 a 44.4% increase at an average 2% per annum | Increase in renewal and maintenance costs Increased demand for recreational areas |
| Demographics | Provision of affordable housing | Between 2020 and 2030, the age structure forecasts for the City of Cockburn indicate a 20.1% increase in population under working age | Greater demand for play equipment and sporting facilities in high growth areas. |
| Urban consolidation | Continual development of Greenfield land | Greenfield land will continue to be developed (expected 2030) and higher density residential developments will take place around Cockburn Central | Increased use of existing reserves and demand for upgrading of reserves to a higher standard |
| Environment | Licence issued from Dept of Water to extract groundwater | Restrictions on water extraction, more use of recycled water or alternative sources | Additional cost in establishment of new systems Change in reserves from traditional green areas to mulched areas and dry open space |

An overall increase in population across the City will raise customer expectations (parks, reserves, playgrounds) with regards to the services they provide. Urban development is becoming higher in density resulting in greater demand for ‘green spaces’. In addition, new residential development lots are reducing in size at the expense of private gardens and back yards with the intent that communities will utilise shared Public Open Space (POS).

Between 2020 and 2030, the age structure forecasts for the City of Cockburn indicate a 20.1% increase in population under working age, a 37.9% increase in population of retirement age, and a 21.8% increase in population of working age.

In 2030 the most populous forecast age group will be 30-34 year olds with 11,999 persons. By 2030 the number of people aged under 5 is forecast to increase by 1678 yet the % number in that age group (7.2%) is lower than that of 2020.

The area with the greatest increase in persons aged 65-85+ years between 2020 and 2030 in the City is Coogee – North Coogee estimated at 14,075 to 19,404 an increase of 38% .

The areas with the greatest increase in persons aged 0 to 15 years are Treeby, North Coogee and South Lake – Cockburn Central.

The suburbs that forecast the greatest increase in number of dwellings are Treeby and North Coogee.

North Coogee is where the SAR (special area rates) parks are located.

## 4.2	Changes in Technology

Notable technology changes are forecast to have an effect on the delivery of services covered by this plan. Even more significantly, changes in energy consumption, environmental impacts (i.e. weather patterns) and water abstraction which will continue to be monitored and addressed in future revisions of this plan. Technology will enhance the delivery of services through better ground water monitoring, application of wetting agent/fertiliser to efficiently use irrigation water.

| Technology Change | Effect on Service Delivery |
| --- | --- |
| Central Irrigation Control | Improve ground water budget recording and forecasting |
| Machinery and Equipment | Improved design with low emission or electrical alternative |
| Lighting –LED | Improve energy efficiency |
| Tree Canopy Mapping | Identify percentage of cover, areas to plant trees and trees health |
| Ground Water Management Software | Ensures water delivered to POS and Streetscapes is tailored to environmental conditions and GW allocations set by DWER. Enhances water budget forecasting and recording for annual reporting. |
| Using online reporting platforms to capture greenhouse gas emissions, energy and water consumption for council buildings, parks and reserves | Greater efficiencies in use and management of resources, improves ease of reporting and greater transparency |
| Using tablets to capture data and respond to customer requests | Increases ability to capture on ground changes and apply management responses-improvement in reserve condition |
| Increased use of use of solar powered lights for POS lighting in specific locations | Decrease in energy consumption |
| Steam trials as a method of weed control | Diversification of weed control approaches - improvement to POS amenity and reduced chemical usage |

### 4.2.1 Alternate Water Supply System (AWSS) Groundwater Interception Device (GID)

Studies confirmed the need to reduce groundwater flow and associated inflow of nutrients to the proposed marina at Port Coogee. Groundwater flow and hydrodynamic modelling concluded that a 90% reduction in flow of groundwater / nutrients was required as excess nitrogen contributed to phytoplankton blooms. The solution was to install a sub-surface permeable cut-off drain to intercept the groundwater flowing into the marina.

The Port Coogee Alternate Water Supply System (AWSS) comprises of:

- Two interception pump stations each with two submersible pumps, these pumps service the irrigation and re-use network storage as well as the reinjection bore;

- Two distribution pump skids drawing water from the storage tank. These pump skids area each servicing separate water distribution schemes, one for the irrigation of public open space, the other for non-potable use by residents and miscellaneous construction purposes;

- The distribution network;

- The reinjection bore.

The AWSS has a separate maintenance plan managed by external Facilities Contractor Advisian which includes various monthly, bi-annual and annual readings including monthly irrigation system readings, annual end of year water readings and bi-annual inspections of water meters. For more information on the AWSS please see Port Coogee Alternative Water Supply Scheme – General Information document.

Figure 4.2.1	8500m3 capacity netork storage tank at Lucretia Park, January 2008

Figure 4.2.2, Lucretia Park, Port Coogee, January 2021

### 4.2.2 Fertigation

Installation of fertigation cabinets at active reserves, these work in conjunction with existing irrigation systems but inject fertilisers into the water whilst it is running.

Liquid wetting agent is also used to lock in the fertiliser reducing wasteful pooling and runoff resulting in improved water retention, penetration and more efficient usage of ground water.

### 4.2.3 Soil Moisture Sensors (SMS)

The City is trialling SMS at Legacy Park with a secondary system planned for Atwell oval. These track soil moisture and movement throughout the soil profile.

Software data from the SMS will be fed into the irrigation system, following a rain event for example; the irrigation output will be reduced based on the stress point, full point and saturation point of the soil enabling more efficient practice.

## 4.3	Demand Management Plan

Demand for new services will be recognised through a combination of managing and upgrading of existing assets and providing new assets. Demand management practices include non-asset solutions, insuring against risks and managing failures.

Demand management strategies provide alternatives to the creation of new assets in order to meet demand and look at ways to modify customer demands so that the utilisation of existing assets is maximised and the need for new assets deferred or reduced. The objective of demand management is to actively seek to modify customer demands for services in order to;

- Optimise the utilisation and performance of existing assets,

- Reduce or defer the need for new assets,

- Meet organisation’s strategic objectives,

- Deliver a more sustainable service, and

- Respond to changing customer needs.

The opportunities identified to date for demand management, the impact these drivers may have on future service delivery and the utilisation of these assets are shown in the Table 4.3.1.

Table 4.3.1	Demand Management Plan Summary

| Service Activity | Demand Management Plan |
| --- | --- |
| Public Open Space Strategy | Strategic planning framework which outlines how the City’s will manage public open space and recreational area and the associated assets for current and future community needs |
| Natural Areas Management Strategy | Strategic planning framework which outlines how the City’s will manage natural areas and the associated assets for the current and future community needs |
| Public Open Space & Natural Areas Maintenance | Annual maintenance plans / programs identify and forecast all maintenance works required within parks and bushlands |
| Sustainability | Sustainability is a key strategic value for the City of Cockburn, embedded in the culture of our organisation. The ‘integrated reporting framework’ embeds sustainability into responsible decision making at all levels. The framework includes the City’s Sustainability Policy, Sustainability Strategy, Action Plan and annual State of Sustainability Report |
| Urban Forest | Strategic approach to improving the canopy cover throughout the City and set direction for the management of streetscapes and public open space to improve liveability and wellbeing |
| Playground Rationalisation Strategy | Review existing framework and develop process for location of units, renewal and demand in growth areas |
| Park Maintenance | Annual maintenance plans / programs identify and forecast all maintenance works required within parks |
| BBQ’s | Review existing framework and develop process for new BBQ’s in existing and growth areas |
| Shade Sails | Continue to implement Councils annual policy on shade sails over playgrounds to meet community expectations |
| Irrigation | Audit of existing infrastructure and review current central control systems to improve licensed ground water management |
| Water Resource Operating Strategy | Strategic framework to manage the abstraction of groundwater to ensure a sustainable resource and mitigate impacts the environment |
| Street Tree Database | Input and extraction of data to the new street tree data base using GIS to manage trees within the streetscape environment |
| Infrastructure location, provision and function | ‘Outdoor spaces’ is a key topic area of the Age-friendly Cities initiative, ensuring sufficient provision of infrastructure that’s functional in design and located with consideration is key to ensuring City open spaces are inclusive. City of Cockburn Age Friendly Strategy 2016 - 202 Global Age Friendly Cities – World Health Organisation (WHO) |

## 4.4	New Assets from Growth

The new assets required to meet growth will be constructed by the City of Cockburn and acquired from land developments. The forecast for capital growth has been taken from the Parks capital works plan for new hard infrastructure.

The forecast for donated assets likely to be received from developers over the next five years has been calculated by averaging out the previous 6 years total donated assets received through subdivisions. This equates to approx. $1.49m per year in hard infrastructure and approx. 19 Ha per year of open space.

Graph 4.4	New Assets from Growth

New asset growth from council has been taken from the Parks & Environment Capital works 10 year plan as shown in Appendix B.

# 5. Lifecycle Management Plan

The lifecycle management area details how the City of Cockburn plans to manage and operate the parks and environment infrastructure assets while optimising lifecycle costs. The data is based on the City’s financial and operational asset registers.

## 5.1 Asset Data

The Parks and Environment data has been split into 3 categories to align with the financial reporting. These are described in the following table:

| Asset Group | Asset Classification |
| --- | --- |
| INFRASTRUCTURE | Amenities |
|  | Bins |
|  | Fences |
|  | Lighting |
|  | Minor structure |
|  | Playground walls |
|  | Signs |
|  | Sporting equipment |
| IRRIGATION | All irrigation equipment |
| PLAYGROUNDS | Playground equipment |
|  | Playground surfaces |
|  | Shade structures |

### 5.1.1	Asset Condition

The Condition profile of the City’s parks and environment assets are measured using a 1 to 5 rating system as outlined below.

| Rating | Description |  |  |
| --- | --- | --- | --- |
| 1 | Excellent |  | A new asset or an asset in overall excellent condition with only a slight condition decline. |
| 2 | Good |  | An asset in an overall good condition but with minor signs of deterioration evident, serviceability may be slightly impaired. Minor maintenance is required. |
| 3 | Moderate |  | An asset with obvious signs of deterioration. Significant maintenance is required. |
| 4 | Poor |  | An asset in a poor condition. Condition deterioration is severe and serviceability is becoming limited. Significant renewal or upgrade is required. |
| 5 | Very poor |  | An asset that has failed and is no longer serviceable. There would be a risk in leaving the asset in service. Replacement is required. |

A Condition profile for the asset categories is shown below.

Graph 5.1.1 Condition Profile for Hard Infrastructure Assets

Table  5.1.1	Summary of Hard Infrastructure Assets – Condition

| Condition | Irrigation | Playgrounds | Infrastructure | All assets |
| --- | --- | --- | --- | --- |
|  | % of asset group |  |  |  |
| 1 | 30.07% | 32.06% | 30.37% | 30.67% |
| 2 | 42.99% | 44.08% | 46.19% | 45.22% |
| 3 | 16.24% | 22.20% | 19.03% | 19.21% |
| 4 | 4.72% | 1.39% | 3.69% | 3.39% |
| 5 | 5.98% | 0.26% | 0.72% | 1.51% |
| Total | 100% | 100% | 100% | 100% |

From the above it can be seen that the majority of assets (75.9%) are in a good to excellent condition.

Of those assets at the intervention level of condition 4 or 5, Infrastructure assets account for 61%, Irrigation 39% and Playgrounds <1%.

A breakdown of the Condition 4 & 5 infrastructure assets by type is shown below:

| Asset Type | Condition 4 | Condition 5 | Total CRC | % of 4 & 5 |
| --- | --- | --- | --- | --- |
| Amenities | $155,500 | $59,000 | $214,500 | 7% |
| Bins | $0 | $2,949 | $2,949 | 0% |
| Fence | $1,527,704 | $176,115 | $1,703,819 | 57% |
| Minor Structure | $543,275 | $145,859 | $689,134 | 23% |
| Public Art | $61,000 | $23,000 | $84,000 | 3% |
| Signs | $110,458 | $57,279 | $167,737 | 6% |
| Sporting Equipment | $19,495 | $1,500 | $20,995 | 1% |
| Lighting | $74,910 | $19,800 | $94,710 | 3% |
| Total | $2,492,342 | $485,502 | $2,977,844 | 100% |

A breakdown of the Condition 4 & 5 irrigation assets by type is shown below:

| Asset Type | Condition 4 | Condition 5 | Total CRC | % of 4 & 5 |
| --- | --- | --- | --- | --- |
| Discharge Assembly | $30,000 | $5,000 | $35,000 | 2% |
| Electrical Cabinets | $46,350 | $20,600 | $66,950 | 3.45% |
| Valves | $48,400 | $62,400 | $110,800 | 5.71% |
| Bores | $107,000 | $20,000 | $127,000 | 6.54% |
| Sprinklers | $153,861 | $158,630 | $312,491 | 16.10% |
| Pumps | $154,500 | $550,900 | $705,400 | 36.34% |
| Infield Pipes | $316,920 | $266,720 | $583,640 | 30.06% |
| Total | $857,031 | $1,084,250 | $1,941,281 | 100% |

A breakdown of the Condition 4 & 5 playground assets by type is shown below:

| Asset Type | Condition 4 | Condition 5 | Total CRC | % of 4 & 5 |
| --- | --- | --- | --- | --- |
| Playgrounds Equipment | $291,217 | $4,244 | $295,461 | 81% |
| Playground Surface | $0 | $54,000 | $54,000 | 15% |
| Shade Poles | $0 | $0 | $0 | 0% |
| Shade Sails | $16,000 | $0 | $16,000 | 4% |
| Water Play Equipment | $0 | $0 | $0 | 0% |
| Total | $307,217 | $58,244 | $365,461 | 100% |

Condition profile of Hard Infrastructure Assets by park hierarchy

The below graph shows the breakdown of asset condition by park hierarchy, it can be seen that the condition of hard infrastructure assets is uniformly apportioned across all park categories. Condition 1 and 2 infrastructure is mostly found in SAR parks which is to expected as these are under 10 years old.

### 5.1.2 Useful life

A useful life has been applied to all hard infrastructure, this is based on the current depreciation rate as used by Finance Services.  The following table shows all asset types included in this AMP and their assigned useful life.

Table 5.1.2A Hard Infrastructure (depreciable assets)

| Asset Group | Asset Classification | Asset Type | Useful Life (years) |
| --- | --- | --- | --- |
| INFRASTRUCTURE INFRASTRUCTURE | Amenities | BBQ – 1 & 2 Plate Bicycle Rack Drink Fountain Flag Pole Picnic Setting Picnic Shelter Seat Shower Tap | 10 15 20 20 20 20 10 25 20 |
|  | Bins | Dog Poo Pouch Stations Rubbish Bins | 15 & 10 13 & 23 |
|  | Boardwalk | Timber Steel | 30 20 |
|  | Bridges (Timber) | Footbridges Decking | 30 30 |
|  | Fence | Baseball Fence Bollard Chain Wire Colorbond Conservation Edging Feral / Rabbit proof Gate Handrail / Railings Retaining Wall Wall | 15 15 & 25 & 40 20 20 25 25 25 15 & 25 25 & 15 40 20 & 40 & 50 |
|  | Lighting | Lumins Poles | 15 25 |
|  | Minor structure | Amphitheatre BMX Facility Courts – Asphalt Courts – Concrete Cricket – Concrete base Discus / shot put throwing circle Gazebo Jetties Lookout Shelter Synthetic surface (bowls, cricket) Skate Facility Tree Protector Under-surface – Asphalt Under-surface – Concrete | 50 15 20 50 50 50 25 25 35 40 20 50 20 20 50 |
|  | Sporting equipment | Basketball / Netball rings Cricket – Poles Cricket – Rubber backing Goals | 10 40 10 15 |
|  | Public Art | Public Art Material Aluminum Bronze Canvas Ceramic Concrete Copper Glass Paint Steel Stone Timber | 40 50 10 40 50 50 40 10 50 50 25 |
|  | Signs | All signs | 15 |
| IRRIGATION | Irrigation | Aerators Bores Cabinet/Electrics/Controller Discharge Assembly Infield Pipes Iron Filtration Unit Pumps Sprinklers Valves | 20 25 15 20 20 30 10 5 15 |
| PLAYGROUNDS | Playgrounds | Exercise equipment Liberty Swing Play equipment Shade poles Shade fabric | 15 15 15 15 5 |

Contrary to the previous PEAMP the infrastructure classification has the most condition 4 and 5 assets. Not all aging infrastructure is located within City parks but those with the highest count of condition 4 and 5 assets are in Table 5.1.2B

Table 5.1.2B Parks with condition 4 and 5 infrastructure

| Park Name and Suburb | Infrastructure | Count |
| --- | --- | --- |
| Bibra Lake Reserve | Amenities | 9 |
| Bibra Lake | Bins | 1 |
|  | Fence | 10 |
|  | Minor Structure | 5 |
| Manning Park Reserve Hamilton Hill | Amenities | 8 |
|  | Fence | 7 |
|  | Minor Structure | 2 |
|  | Public Art | 1 |
| Coogee Beach Reserve North Coogee | Amenities | 3 |
|  | Bins | 1 |
|  | Fence | 2 |
|  | Minor Structure | 3 |
|  | Sporting Equipment | 8 |
| C Y O'Connor North Coogee | Amenities | 10 |
|  | Bins | 1 |
|  | Minor Structure | 4 |

Minor works are planned to upgrade aging park infrastructure in addition, more long-term projects such as the Bibra Lake Master Plan which will see revegetation, signage upgrades and installation of art work.

### 5.1.3 Asset valuations

The value of assets as covered by this AMP are summarised in Table 5.1.3, these were last re-valued as part of the City’s revaluation process in June 2020.

Table 5.1.3A	Parks Infrastructure Current Asset Values

| Park Hierarchy | Infrastructure $m | Irrigation $m | Playgrounds $m | Current Replacement Cost $m |
| --- | --- | --- | --- | --- |
| Bushland | 16.01 | 0.89 | 1.30 | 18.20 |
| Community | 2.05 | 0.68 | 0.70 | 3.43 |
| Major passive | 3.03 | 0.41 | 2.64 | 6.07 |
| Minor Passive | 25.74 | 10.03 | 13.25 | 49.02 |
| Playing Field | 12.46 | 3.33 | 3.15 | 18.94 |
| SAR Park | 2.75 | 1.10 | 0.82 | 4.67 |
| Streetscape | 5.46 | 1.71 | 0.23 | 7.40 |
| Total | $67,497,583 | $18,144,416 | $22,082,656 | $107,724,655 |

Table 5.1.3B Average value of hard infrastructure per hectare by park hierarchy

| Park Hierarchy | CRC of all assets | Hectares | CRC of irrigation per Ha | CRC of Infrastructure per Ha |
| --- | --- | --- | --- | --- |
| Bushland | $18,204,498 | 1796.58 | $493 | $8,914 |
| Community Facility | $3,425,439 | 99.80 | $6,769 | $20,533 |
| Major passive | $6,072,389 | 45.29 | $9,036 | $66,830 |
| Minor Passive | $49,016,979 | 520.80 | $19,256 | $49,427 |
| Playing Field | $18,937,219 | 231.90 | $14,371 | $53,725 |
| SAR Parks - OP | $4,667,653 | 16.70 | $65,976 | $164,405 |
| Streetscape | $7,400,476 | 489.72 | $3,494 | $11,151 |
| Total | $107,724,655 | 3200.79 |  |  |

## 5.2 Maintenance and Operating expenditure

Maintenance work includes reactive or planned maintenance work activities.

Reactive maintenance is unplanned repair work, carried out in response to service requests, from Management or Supervisory directions.

Planned maintenance is work that is identified and managed through a maintenance schedule, these activities include inspection, assessing the condition against failure or breakdown experience, prioritising, scheduling and reporting along with capture of rectification works to develop a maintenance history and improve maintenance and service delivery performance.

As well as general park maintenance planned maintenance activities include:

Street tree management – 	Pruning of approximately 1500 street trees under power lines

Road Reserve Maintenance – Mowing of road reserves and spraying of footpath and kerb lines.

Streetscape maintenance –	Maintaining landscaped streetscapes

Cyclic maintenance is replacement of higher value components/sub-components of assets that is undertaken on a regular cycle including irrigation and playground repairs. This work generally falls below the capital/maintenance threshold.

Graph 5.2.1 Maintenance breakdown by Park Hierarchy

Table 5.2.1 	Maintenance Expenditure Trends – Parks

| Year | Maintenance Expenditure |  |  | Operating Expenditure | Budget | Total Expenditure |
| --- | --- | --- | --- | --- | --- | --- |
|  | Planned | Reactive | Reactive% |  |  |  |
| 2013-14 | $9,634,505 | $935,698 | 9% | $566,060 | $10,345,571 | $11,136,263 |
| 2014-15 | $9,991,443 | $1,188,913 | 11% | $452,991 | $11,300,747 | $11,633,347 |
| 2015-16 | $10,162,412 | $1,137,325 | 10% | $464,128 | $11,819,096 | $11,763,865 |
| 2016-17 | $11,543,800 | $1,269,232 | 10% | $489,771 | $12,308,220 | $13,302,803 |
| 2017-18 | $10,841,729 | $1,132,495 | 9% | $675,221 | $12,882,874 | $12,649,445 |
| 2018-19 | $11,283,033 | $1,224,167 | 10% | $798,648 | $13,509,434 | $13,305,848 |
| 2019-20 | $12,411,063 | $1,282,769 | 9% | $659,113 | $14,126,764 | $14,352,945 |
| Budget for 2020-21 |  |  |  |  |  |  |
|  |  |  |  |  | $14,674,487 |  |

The following graphs show the last 5 years maintenance expenditure against irrigation, playgrounds, street trees, landscape maintenance and mowing split between planned and reactive maintenance.

Irrigation maintenance expenditure

Playground maintenance expenditure

Street Tree maintenance expenditure

Landscape maintenance expenditure

Mowing maintenance expenditure

Maintenance expenditure levels are considered to be inadequate to meet required service levels.  Future revision of this asset management plan will include linking required maintenance expenditures with required service levels.

Assessment and prioritisation of reactive maintenance is undertaken by Council staff using experience and judgement.

Reactive maintenance is carried out in accordance with response levels of service detailed in Appendix C.

Table 5.4.2	Maintenance Expenditure Trends – Environment

| Year | Maintenance Expenditure |  |  | Operating Expenditure | Budget | Total |
| --- | --- | --- | --- | --- | --- | --- |
|  | Planned | Reactive | Reactive% |  |  |  |
| 2013-14 | $853,679 | $976,544 | 53% | $1,040,798 |  | $2,871,021 |
| 2014-15 | $964,077 | $1,041,686 | 52% | $856,132 | $3,035,143 | $2,861,895 |
| 2015-16 | $1,022,617 | $1,187,049 | 54% | $803,363 | $3,168,862 | $3,013,029 |
| 2016-17 | $987,006 | $1,236,234 | 56% | $1,025,933 | $3,213,664 | $3,249,173 |
| 2017-18 | $946,137 | $1,178,784 | 55% | $1,032,393 | $3,368,955 | $3,157,314 |
| 2018-19 | $1,160,196 | $1,266,101 | 52% | $1,181,303 | $4,511,789 | $3,607,600 |
| 2019-20 | $1,333,733 | $1,851,875 | 58% | $1,171,630 | $4,923,599 | $4,357,238 |

### 5.2.1 Standards and specifications

Renewals, upgrades and maintenance work are carried out in accordance with the following Standards and Specifications.

- AS 4685 Playground Equipment – General safety requirements

- AS 4422 Playground Surfacing – Specifications and Requirements

- AS 4486 Playgrounds and Play Equipment – Development, installation, inspection, maintenance and operation

- Main Roads WA – Traffic Management guidelines

- Main Roads Landscape specification

- Main Roads Utilities code of Practice

- AS 4373-2007 Pruning of Amenity Trees

- Code of Practice for the Electrical Safety for Vegetation Control Work near Live Power Lines

- City of Cockburn firebreak notice

- City of Cockburn Administration Policy – AEW1 Street Verge Improvements

- City of Cockburn Administration Policy – AEW4 Installation of Playgrounds/ Recreational Equipment on Reserves

- City of Cockburn Administration Policy – AEW7 Establishment of Community Gardens

- City of Cockburn Strategic Policies – SEW1 Maintenance of verges / Public Open Spaces following residential subdivisions

- City of Cockburn Strategic Policies – SPD1 Bushland Conservation Policy

- City of Cockburn Strategic Policies – SPD4 Liveable Neighbourhoods

- City of Cockburn Strategic Policies – SPD5 Wetland Conservation Policy

- City of Cockburn Strategic Policies – SES3 Evaluation of Tenders

- City of Cockburn Position Statements – PSEW10 Graffiti & vandalism Response – Council Property

- City of Cockburn Position Statements – PSEW15 Removal and Pruning of Trees

- City of Cockburn Position Statements – PSEW16 Unkempt Verge mowing

- City of Cockburn Position Statements – PSEW17Outdoor Sports Lighting on Recreation Reserves

- City of Cockburn Position Statements – PSEW18 Trees on Privately Owned Land

- City of Cockburn Position Statements – PSEW19 Shade to Playgrounds on Recreation Reserves

- City of Cockburn SC39 Asset management

- City of Cockburn Disability Access and Inclusion Plan

- AS 26983-1990 Plastic pipes and fittings to irrigation and rural applications.

- City of Cockburn – Greenhouse Gas Emissions Reduction Strategy 2011-2020

- Natural Area management Strategy 2012 -2022

- Engineering Subdivision Guidelines – Revegetation completion criteria

### 5.2.2 Summary of future maintenance expenditures

The average operational and maintenance expenditure per hectare of park is currently $10,221 (excluding SAR parks) for parks and $2425 per hectare of bushland. The forecast maintenance expenditure has been based on the new assets from growth figure of 19ha of new park per year multiplied by the current expenditure per hectare.

Future maintenance expenditure is forecast to trend in line with the value of the asset stock.  Note that all costs include 2% CPI increase per year forward.

Graph 5.2.2 Forecast Maintenance Expenditure

Deferred maintenance, ie works that are identified for maintenance and unable to be funded are to be included in the risk assessment process in the infrastructure risk management plan.

Maintenance is funded from Council’s operating budget and grants where available. New assets and services are to be funded from Council’s capital works program and grants where available. This is further discussed in Section 6.2.

## 5.3 Renewal/Replacement Plan

Renewal expenditure is major work which does not increase the asset’s design capacity but restores, rehabilitates, replaces or renews an existing asset to its original service potential.  Work over and above restoring an asset to original service potential is upgrade/expansion or new works expenditure.

Growth has not been included in any of the renewal graph calculations. A 2% CPI increase per year forward has been added to all projected renewal graphs. The expenditure gap is further analysed in Section 6.2.

### 5.3.1 Playgrounds

Playground Renewals are based on a useful life of 15 years for all playground and fitness equipment. The projected replacement year has been taken from the 15 year replacement program currently used by the Parks service unit and based upon annual audit data. These are detailed in Appendix D along with the playgrounds requiring replacement within the next 10 years. All figures include a 2% CPI increase per year forward.

Graph 5.3.1 	Preliminary Renewals – Playgrounds

### 5.3.2 Irrigation

Irrigation renewals are based on age and useful life as identified in section 5.1. There are currently $0.2m of irrigation assets that are considered to be past their projected renewal date and these form the basis for the Preliminary 6 Year Renewal Program as shown in Appendix E.  A 2% CPI increase has been added per year forward.

Graph 5.3.2 Projected Renewals – Irrigation

### 5.3.3 Infrastructure

Infrastructure assets requiring renewal have been identified from estimates of condition and remaining life as obtained from the asset register. An asset intervention level of condition 4 or 5 has been set and this formed the basis of the current program as shown in Appendix F. All figures include a 2% CPI increase per year forward.

Graph 5.3.3 Projected Renewals – Infrastructure

Renewal will be undertaken using ‘low-cost’ renewal methods where practical. The aim of ‘low-cost’ renewals is to restore the service potential or future economic benefits of the asset by renewing the assets at a cost less than replacement cost.

### 5.3.4 Renewal standards

Renewal work is carried out in accordance with the Standards and Specifications outlined in Table 5.2.1.

### 5.3.5 Summary of future renewal expenditure

Projected future renewal expenditures are forecast to increase over time as the asset stock ages.

The preliminary capital renewal projects are shown in Appendices D, E and F.

Renewals are to be funded from Council’s capital works program and grants where available.  This is further discussed in Section 6.2.

## 5.4 New/upgrade plan

New works are those works that create a new asset that did not previously exist, or works which upgrade or improve an existing asset beyond its existing capacity.  They may result from growth, social or environmental needs.  Assets may also be acquired at no cost to the Council from land development.  These assets from growth are considered in Section 4.4.

### 5.4.1 Selection criteria

Considerable planning has gone into proposals to improve the City’s parks and open spaces. In recent years the City has adopted the Manning Park Master Plan, Bibra Lake Master Plan and the Coogee Beach Master Plan. These are all major projects with years of planning behind them, as funding is limited from external sources, project prioritisation and staging is required.

### 5.4.2 Summary of future upgrade/new assets expenditure

Planned upgrade/new asset expenditures are summarised below in Graph 6.1.1.  All costs include a 2% CPI increase per year forward.

## 5.5 Disposal Plan

Disposal includes any activity associated with disposal of a decommissioned asset including sale, demolition or relocation.  There are no disposals planned for any Parks and Environment Infrastructure assets at this time.

# 6. Financial Analysis

The Financial Analysis section of this report provides the recommended financial forecasts for the next 10 years. This section brings together the various types of expenditure described throughout the previous sections of the AMP and provides recommended budgets for Council to achieve the appropriate level of service through municipal funding.

## 6.1	Financial Statements and Projections

From the financial asset register, the value of assets as covered by this asset management plan are summarised in Table 6.1.1 Current Replacement Cost and Depreciation. The current replacement cost, fair value (also known as written down value or depreciated replacement cost), depreciation and the annual depreciation values are shown.

Table 6.1.1A 	Current replacement cost and depreciation

| Asset Classification | Current Replacement Cost | Fair value | Annual Depreciation Expense |
| --- | --- | --- | --- |
| Amenities | $7,886,395 | $5,327,559 | $628,077 |
| Bins | $731,177 | $561,564 | $39,035 |
| Fencing | $32,953,398 | $21,548,170 | $1,160,218 |
| Lighting | $5,525,075 | $3,961,355 | $280,770 |
| Minor structure | $14,906,599 | $10,797,369 | $380,838 |
| Public Art | $1,509,500 | $1,142,974 | $15,095 |
| Signs | $1,586,544 | $994,525 | $105,770 |
| Sporting Equipment | $2,398,895 | $1,768,700 | $127,324 |
| Total Infrastructure | $67,497,583 | $46,102,215 | $2,737,125 |
| Irrigation | $18,144,416 | $11,595,041 | $640,559 |
| Playground | $22,082,656 | $14,950,267 | $1,501,320 |
| TOTAL ALL | $107,724,655 | $72,647,523 | $4,879,003 |

Table 6.1.1B 	Current replacement cost and depreciation based on hierarchy

| Park Hierarchy | Current Replacement Cost | Fair value | Annual Depreciation Expense |
| --- | --- | --- | --- |
| Bushland | $18,204,498 | $12,111,572 | $737,997 |
| Community | $3,425,439 | $2,388,467 | $152,609 |
| Major Passive | $6,072,389 | $4,111,641 | $339,925 |
| Minor Passive | $49,016,979 | $33,215,474 | $2,338,454 |
| Playing Field | $18,937,219 | $12,567,603 | $857,234 |
| SAR Parks | $4,667,653 | $3,668,692 | $191,949 |
| Streetscapes | $7,400,476 | $4,584,075 | $260,835 |
| TOTAL | $107,724,655 | $72,647,523 | $4,879,003 |

The financial projections are shown in Graph 6.1.1 for forecasted operating (operations and maintenance) and capital expenditure (renewal and upgrade/expansion/new assets) for all Parks and Environment assets.

All projection figures include a 2% CPI increase per year forward.

Graph 6.1.1	Forecasted Operating and Capital Expenditure

The following tables show the expenditure gap between projected renewals and the budget allocation for each asset group. The projected renewals are taken from the renewals graphs as shown in section 5.3 and detailed in the Appendices.

Table 6.1.1C Playgrounds – Preliminary renewals and budget allocation gap

| YEAR | PROJECTED RENEWAL | PROPOSED BUDGET ALLOCATION from LTFP | FUNDING GAP | CUMULATIVE GAP |
| --- | --- | --- | --- | --- |
|  | (inc CPI) |  |  |  |
| 2020-21 | $297,569 | $252,933 | $44,635 | -$44,635 |
| 2021-22 | $591,293 | $532,163 | $59,129 | -$103,765 |
| 2022-23 | $744,119 | $706,913 | $37,206 | -$140,971 |
| 2023-24 | $654,655 | $654,655 | $0 | -$140,971 |
| 2024-25 | $527,456 | $527,456 | $0 | -$140,971 |
| 2025-26 | $565,783 | $565,783 | $0 | -$140,971 |
| 2026-27 | $613,705 | $613,705 | $0 | -$140,971 |
| 2027-28 | $470,694 | $470,694 | $0 | -$140,971 |
| 2028-29 | $651,563 | $651,563 | $0 | -$140,971 |
| 2029-30 | $619,088 | $619,088 | $0 | -$140,971 |
| TOTAL | $5,735,925 | $5,594,954 | $-140,971 |  |

The 10 year cumulative funding gap for playground renewals is a deficit of $140,971.

Table 6.1.1 D Irrigation – Projected renewals and budget allocation gap

| YEAR | PROJECTED RENEWAL (inc CPI) | PROPOSED BUDGET ALLOCATION from LTFP | FUNDING GAP | CUMULATIVE GAP |
| --- | --- | --- | --- | --- |
| 2020-21 | $507,343 | $431,242 | $76,102 | $76,102 |
| 2021-22 | $688,208 | $631,525 | $56,683 | $132,785 |
| 2022-23 | $701,973 | $666,874 | $35,099 | $167,883 |
| 2023-24 | $465,123 | $465,123 | $0 | $167,883 |
| 2024-25 | $474,425 | $474,425 | $0 | $167,883 |
| 2025-26 | $589,494 | $589,494 | $0 | $167,883 |
| 2026-27 | $601,284 | $601,284 | $0 | $167,883 |
| 2027-28 | $218,977 | $218,977 | $0 | $167,883 |
| 2028-29 | $223,357 | $223,357 | $0 | $167,883 |
| 2029-30 | $1,884,826 | $1,884,826 | $0 | $167,883 |
| TOTAL | $6,355,010 | $6,187,127 | $-167,883 |  |

The 10 year cumulative funding gap for irrigation is a deficit of $167,883.

Table 6.1.1E Infrastructure – Projected renewals and budget allocation gap

| YEAR | PROJECTED RENEWAL | PROPOSED BUDGET ALLOCATION | FUNDING GAP | CUMULATIVE GAP |
| --- | --- | --- | --- | --- |
|  | inc CPI | from LTFP |  |  |
| 2020-21 | $48,245 | $41,008 | $7,237 | $7,237 |
| 2021-22 | $172,788 | $155,509 | $17,279 | $24,516 |
| 2022-23 | $169,560 | $161,082 | $8,478 | $32,994 |
| 2023-24 | $142,463 | $142,463 | - | $32,994 |
| 2024-25 | $1,243,929 | $1,243,929 | - | $32,994 |
| 2025-26 | $261,086 | $261,086 | - | $32,994 |
| 2026-27 | $2,646,852 | $2,646,852 | - | $32,994 |
| 2027-28 | $1,797,584 | $1,797,584 | - | $32,994 |
| 2028-29 | $1,622,156 | $1,622,156 | - | $32,994 |
| 2029-30 | $1,039,655 | $1,039,655 | - | $32,994 |
| Total | $9,144,317 | $9,111,323 | -$32,994 |  |

The 10 year funding gap for infrastructure is a deficit of $32,994.

## 6.2 Funding Strategy

Projected expenditure identified in Section 6.1 is to be funded from Council’s operating and capital budgets.

The funding strategy is detailed in the City’s 10 year Long Term Financial Plan and has been added to this plan in Tables 6.1.1C, 6.1.1D and 6.1.1E.

## 6.3 Sustainability of Service Delivery

The asset management framework is part of the of the Western Australian Government’s Local Government Reform Program. The aim of the framework is to enhance the sustainable management of local government assets by encouraging ‘whole of life’ and ‘whole of organisation’ approaches and the effective identification and management of risks associated with the use of the assets.

There are three key performance indicators for financial sustainability as recommended in the National Framework guidelines that have been considered in the analysis of the Parks and Environment infrastructure financial data and these are outlined below.

### 6.3.1 Asset Consumption Ratio

This ratio shows the written down current value of the City’s depreciable Parks & Environment assets relative to their ‘as new’ value in up to date prices.

It is calculated by dividing the written down value (WRV) by the current replacement cost. (CRC). These figures are shown in Table 6.1.1.

The ratio highlights the aged condition of the assets.

The standard is met if the ratio can be measured and is 50% or greater.

The target ratio should be between 50% and 75%. A ratio of less than 50% indicates a rapid deterioration of the asset base, whilst a ratio greater than 75% may indicate an over investment in the asset base.

The ratios calculated show that the City is adequately investing in all of Parks assets at this time.

| Asset | Consumption Ratio |  |  |  | Standard achieved |
| --- | --- | --- | --- | --- | --- |
|  | 2012-13 | 2013-14 | 2016-17 | 2019-20 |  |
| Infrastructure | 64.20% | 69.40% | 69.28% | 68.30% | Improving |
| Irrigation | 53.02% | 61.56% | 60.82% | 63.90% | Improving |
| Playgrounds | 68.62% | 66.26% | 57.34% | 67.70% | Improving |
| ALL ASSETS | 61.80% | 66.05% | 64.31% | 67.40% | Improving |

Integrated Planning and Reporting Advisory Standard KPI targets are outlined below.

Standard is not met if ratio data cannot be identified or ratio is less than 50%.

Standard is met if ratio data can be identified and ratio is 50% or greater.

Standard is improving if this ratio is between 60% and 75%.

### 6.3.2 Asset Sustainability Ratio

This ratio indicates whether assets are being replaced or renewed at the same rate that the overall asset stock is wearing out.

It is calculated by dividing the annual capital expenditure spent on replacement and renewals by the annual depreciation expense. If capital expenditure on renewing or replacing assets is at least equal to depreciation on average over time, then the value of the existing stock will be maintained. If capital expenditure on existing assets is less than depreciation then underspending on renewal or replacement of assets will occur and this is likely to result in additional maintenance costs for assets that have exceeded their useful life, these costs may exceed those of renewal or replacement.

This ratio can only be measured accurately if an assessment is made of the total amount spent on capital renewal and replacement. The City does not presently undertake an accurate breakdown of it’s upgrade expenditure and the portion of this that would be replacement is not known and has therefore not been considered. A breakdown of upgrade expenditure is part of the improvement strategy.

The target ratio should be between 90% - 110%. A ratio of less than 90% may indicate an under investment in the renewal and replacement of assets.

The forecast asset sustainability ratios shown below have been calculated on an accumulative basis.

| Asset | Forecast Asset Sustainability Ratio (accumulative) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2020-21 | 2021-22 | 2022-23 | 2023-24 | 2024-25 | 2025-26 | 2026-27 | 2027-28 | 2028-29 | 2029-30 |
| Infrastructure | 1% | 3% | 4% | 4% | 11% | 11% | 21% | 25% | 28% | 28% |
| Irrigation | 37% | 44% | 48% | 45% | 44% | 44% | 44% | 41% | 38% | 48% |
| Playgrounds | 16% | 25% | 31% | 33% | 33% | 33% | 33% | 32% | 33% | 33% |
| ALL ASSETS | 13% | 18% | 21% | 21% | 24% | 24% | 29% | 30% | 31% | 33% |

The above ratios indicate that future forecast renewal funding as outlined in the LTFP and as shown in the funding gap tables in section 6.1 may not be sufficient in the longer term to maintain the Parks and Environment infrastructure assets to an acceptable level.

Integrated Planning and Reporting Advisory Standard KPI targets are outlined below.

Standard is not met if ratio data cannot be identified or ratio is less than 90%.

Standard is met if ratio data can be calculated and ratio is 90%.

Standard is improving if this ratio is between 90% and 110%

### 6.3.3 Asset Renewal Funding Ratio

This is an indicator as to the ability of the City to fund the projected asset renewals and replacements in the future. The Long Term Financial Plan should make annual provisions for renewing assets where their condition has degraded below the intervention level. This requirement will vary from year to year. This ratio can’t be accurately determined without these projections.

The ratio is calculated by dividing the planned capital expenditure (from the long term financial plan) on renewals over the next 10 years by the required (projected) capital expenditure on renewals over the same period.

The standard is met if the ratio is between 75% and 95%.

The forecast asset renewal funding ratios shown below have been calculated on an accumulative basis.

| Asset | Forecast Asset Renewal Funding Ratio |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2020-21 | 2021-22 | 2022-23 | 2023-24 | 2024-25 | 2025-26 | 2026-27 | 2027-28 | 2028-29 | 2029-30 |
| Infrastructure | 85% | 89% | 92% | 94% | 98% | 98% | 99% | 99% | 100% | 100% |
| Irrigation | 85% | 79% | 88% | 80% | 77% | 74% | 75% | 69% | 64% | 81% |
| Playgrounds | 85% | 88% | 91% | 94% | 95% | 96% | 96% | 97% | 97% | 97% |
| ALL ASSETS | 85% | 83% | 90% | 87% | 88% | 87% | 89% | 88% | 87% | 93% |

The ratio shows that in the long term, if the renewal budget is as per the LTFP then the forecast expenditure will be more than adequate to fund the projected renewals for Parks & Environment hard infrastructure assets.

Integrated Planning and Reporting Advisory Standard KPI targets are outlined below.

Standard is not met if ratio data cannot be identified or ratio is less than 75%

Standard is met if the ratio is between 75% and 95%.

Standard is improving if the ratio is between 95% and 105% and the ASR falls within the range 90% to 110% and ACR falls within the range of 50% to 75%.

## 6.4 Valuation Forecasts

Asset values are forecast to increase as additional assets are added to the asset stock from construction and acquisition by Council and from assets constructed by land developers and others and donated to Council.  Graph 6.4.1 shows the projected asset values (CRC) over the planning period including a 2% CPI increase per year forward.

Graph 6.4.1	Projected depreciated replacement cost – Hard Infrastructure (WDV)

The depreciated replacement cost (current replacement cost less accumulated depreciation) will vary over the forecast period depending on the addition of new assets, disposal of old assets and consumption and renewal of existing assets.  Forecast of the assets’ depreciated replacement cost is also shown in Graph 6.4.1.

Graph 6.4.2	 Projected depreciation expense – Hard Infrastructure

Depreciation expense values are forecast in line with asset values as shown in Graph 6.4.2 and include a 2% CPI increase per year forward.

## 6.5 Key Assumptions made in Financial Forecasts

This section details the key assumptions made in presenting the information contained in this asset management plan and in preparing forecasts of required operating and capital expenditure and asset values, depreciation expense and carrying amount estimates.  It is presented to enable readers to gain an understanding of the levels of confidence in the data behind the financial forecasts.

Key assumptions made in this asset management plan are:

- Data supplied was as accurate as possible at the time of compilation of this asset management plan.

- The breakdown of the reactive, planned and operational expenditure is mainly assumed and not considered accurate.

Accuracy of future financial forecasts may be improved in future revisions of this asset management plan by the following actions:

- The improvement of reporting from the Technology One Financials and the implementation of the upcoming Strategic Asset Management (SAM) System.

# 7.	Asset Management Practices

## 7.1	Accounting/Financial Systems

### 7.1.1 Summary of Accounting & Financial Systems

Technology One Financials version 11.09.19.011

### 7.1.2 Accountabilities and Responsibilities for Financial System

Financial Services – for the accounts and costing methodologies

### 7.1.3 Accounting Standards / Regulations / Guidelines

- Australian Accounting Standards including:

- AASB116 – Property, Plant and Equipment

- AASB13 Fair Value Measurement

- AASB136 – Impairment of Assets

- AASB 140 Investment Property

- AASB 5 Non-current Assets Held for Sale and Discontinued Operations

- Local Government Act 1995

- Local Government (Financial Management) Regulations 1996

- Local Government (Functions & General) Regulations 1996

## 7.2 Asset Management Systems (EAM)

### 7.2.1 Summary of Asset Management System

Technology One Enterprise Asset Management version 11.09.19.011

Technology One Intramaps 8.1

### 7.2.2 Summary of how the Enterprise Asset Management System aligns to the 	Accounting / Financial system

The operational asset register within the Enterprise Asset Management system acts as the master asset dataset for determining renewal projections, future refurbishment.

The financial registers values are updated yearly from the operational asset register as part of Assets Services revaluation procedures.

### 7.2.3 Accountabilities and Responsibilities for AM System (s)

Project & Asset Services is accountable and responsible for the EAM system, with other service areas assisting with the currency and maintenance of the data sets within the system databases.

### 7.2.4 Changes to the Asset Management Systems resulting from the AMP

All proposed/agreed system changes will be documented in Section 8 Plan Improvement and Monitoring.

## 7.3	Information Flow Requirements and Processes

The key information flows into this asset management plan are:

- The asset register data on size, age, value, remaining life of the network;

- The unit rates for categories of work/material;

- The adopted service levels;

- Projections of various factors affecting future demand for services;

- Correlations between maintenance and renewal, including decay models;

- Data on new assets acquired by council.

The key information flows from this asset management plan are:

- The assumed Works Program and trends;

- The resulting budget, valuation and depreciation projections;

- The useful life analysis.

These will impact the Long Term Financial Plan, Strategic Community Plan, annual budget and departmental business plans and budgets.

## 7.4	Standards and Guidelines

Asset Management Policy Statement (SC 39) 2017

# 8.	Plan Improvement and Monitoring

## 8.1	Performance Measures

The effectiveness of the asset management plan can be measured in the following ways:

- The degree to which the required cash flows identified in this asset management plan are incorporated into council’s long term financial plan and Strategic Management Plan;

- The degree to which 1-5 year detailed works programs, budgets, business plans and organisational structures take into account the ‘global’ works program trends provided by the asset management plan.

- The degree to which existing and projected service levels and consequences, risks and residual risks are incorporated into Council’s plans.

## 8.2	Improvement Strategy

A key component of the Asset Management Planning Framework is monitoring and reporting improvements.

To reflect the City’s ongoing progress whilst ensuring appropriate ownership and action is maintained the Improvement Strategy has been split into the following sections.

Table 8.2.1	Improvement strategy from 2013 - 2017 PEAMP

| Section | Project | Responsibility | Task | Status |
| --- | --- | --- | --- | --- |
| 2 | Definition and measurement of non-depreciable assets | Asset Services | - Addition of irrigated areas to GIS system | Completed |
| 3.3 | Monitoring performance measures against levels of service | Asset Services & Parks Services | - Investigate customer request configuration - Link budget allocation to levels of service - Service Level Analysis Workshops to review service delivery | Completed |
| 3.5 | Service performance deficiencies | Asset Services & Park Services | - Reassess performance deficiencies and assign priority ratings | Completed |
| 5.1 | Clarification of asset ownership (ie. car parks, footpaths and lighting) | Asset Services | - As part of the integration between Tech one and Intramaps - clearly define a single point of responsibility for each asset classification | Completed |
| 5.1 | Addition of limestone fire access tracks into asset register | Asset services | - New Ospec classification to be created for access tracks to contain firebreaks, etc. - Limestone firebreaks to be added to 2012 intramaps Ospec layer - Firebreaks to be included in next AMP | Ongoing |

Table 8.2.1	Improvement strategy from 2017 - 2020 PEAMP

| Section | Project | Responsibility | Task | Timeline |
| --- | --- | --- | --- | --- |
| 3.3 | Monitoring performance measures against levels of service | Asset Services | - Review historic service level data prior to financial year commence | Completed |
| 3.4 | Risk Management | Parks & Environment Services | - Update risk register | Ongoing annually |
| 5.1 | Improve Asset register | Asset Services | - Minimize gaps in Parks Asset register through monthly QA process | Ongoing monthly process |
| 5.1 | Improve mobility solution | Asset Services | - Mobility solution deploy for Bushland crew - Adding functionalities for Irrigation crew | Completed |
| 5.1 | Asset data pick up and audits across Parks and Reserves Assets | Asset Services & Park Services | - Physical and condition assessment of Parks and Reserves assets | Completed |

The asset management improvement plan generated from this asset management plan is shown in table 8.2.2.

Table 8.2.2	Improvement Strategy 2020 – 2024 PEAMP

| Section | Project | Responsibility | Task | Timeline |
| --- | --- | --- | --- | --- |
| 3.1 | Improve the Customer Request System for better reporting on the City’s Levels of Service | Property & Asset Services & Business Systems | - Improve CRM system | 2022/23 |
| 3.3 | Monitoring performance measures against levels of service | Property & Asset Services | - Review historic service level data prior to financial year commence | 2022/23 |
| 5.1 | Improve asset register and new asset handover process | Property & Asset Services | - Minimize gaps in Parks Asset register | 2022/23 |
| 5.1 | Asset data pick up and audits across Parks and Reserves Assets | Property & Asset Services & Environment, Parks and Landscape | - Physical and condition assessment of Parks and Reserves assets | 2022/23 |
| 5.1 | Asset Data Improvements | Property & Asset Services | - Register wide cleansing of Parks, Park Names and all associated infrastructure | 2021/22 |
| 6.3 | Asset Renewals | Property & Asset Services | - Implementation of Strategic Asset Management (SAM) for improved renewal forecasting | 2021/22 |

## 8.3	Monitoring and Review Procedures

The PEAMP forms part of the City’s Strategic Asset Management Planning Framework (SAMPF), covers four financial years (20/21 – 23/24) and acts as an informing strategy to the City’s Corporate Planning Framework.

Future iterations of the PEAMP will be developed every 4 years and be subject to a 2 year desktop review. The PEAMP review will focus on core elements required by the LTFP, for example asset valuations, growth projections, financial analysis including operating, sustainability ratios and 10 year renewals. This will ensure that future revisions of the LTFP will be derived from a structured AMP development cycle which has received Executive and or Council approval, increasing confidence and integration of asset management data and methodologies into the City’s long term financial planning.

The following diagram provides a visual representation and timelines of the Strategic Asset Management Planning Framework.

The formalisation and alignment of the City’s SAMPF (Asset Management Policy, Strategy and AMP’s) within the Integrated Corporate Planning Framework reflects the City’s increasing maturity and recognises the importance of Asset Management in supporting the City in delivering long term financial sustainability of services and capital asset renewal.

Supported by the relevant business area and the Asset Management Sections of the Property and Assets Business Unit, the Asset Manager has overall responsibility and management for each of the Improvement Strategies identified within section 8 of the PEAMP.

# References

City of Cockburn – Asset Management Strategy 2017 – 2024

City of Cockburn – Strategic Community Plan 2020 – 2030

City of Cockburn – Long Term Financial Plan 2020-2021 to 2029-2030

City of Cockburn – Management Budget 2020 - 2021

City of Cockburn – Enterprise Risk Management

City of Cockburn – Community, Sport and Recreation Facilities Plan 2018 - 2033

City of Cockburn - A Plan for the District 2010 - 2020

City of Cockburn – Annual Business Plan 2017 - 2018

City of Cockburn - MARKYT – Community Scorecard 2017, 2018, 2019, 2020

City of Cockburn - MARKYT – Business Scorecard 2017, 2019

City of Cockburn – Population Forecast – https://forecast.id.com.au/Cockburn

City of Cockburn - Population forecast -  https://forecast.id.com.au/cockburn/population-age-structure

IPWEA, 2011, ‘International Infrastructure Management Manual’, Institute of Public Works Engineering Australia, Sydney, www.ipwea.org.au

Government of Western Australia, Department of The Premier and Cabinet - Western Australian Legislation - Acts in force

The Local Government and Municipal Knowledge Base - LGAM Knowledge Base

Port Coogee Alternative Water Supply Scheme – General Information

Advisian Water System Proposal

# Appendices

## Appendix A	Legislative requirements

| Legislation | Requirement |
| --- | --- |
| Local Government Act 1995 | Provides for a system of Local Government by describing the functions of and providing a framework for the administration and financial management of Local Governments. |
| Environment Protection Act 1986 | Provides for an Environmental Protection Authority for the prevention, control and abatement of pollution and environmental harm, for the conservation, preservation, protection, enhancement and management of the environment. |
| Environment Protection and Biodiversity Conservation Act 1999 | Provides a legal framework to protect and manage nationally and internationally important flora, fauna, ecological communities and heritage places, defined in the Act as matters of national environmental significance. |
| Contaminated Sites Act 2003 | Provides for the identification, recording, management and remediation of contaminated sites. |
| Conservation and Land Management Act 1984 | Makes better provision for the use, protection and management of certain public lands and waters and the flora and fauna thereof, establishes authorities to be responsible therefore, and for incidental or connected purposes. |
| Soil and Land Conservation Act 1945 | Relates to the conservation of soil and land resources, and to the mitigation of the effects of erosion, salinity and flooding. |
| Heritage of Western Australia Act 1990 | Provides for and encourages the conservation of places which have significance to the cultural heritage of the state. |
| Rights in Water and Irrigation Act 1914 | Makes provisions for the regulation, management, use and protection of water resources to provide for irrigation purposes. |
| Biological Control Act 1986 | Makes provision for the Biological Control of Pests. |
| Poisons Act 1964 | Regulates and controls the possession, sale and use of poisons. |
| Wildlife Conservation Act 1950 | Provides for the conservation and protection of native flora and fauna. |
| Dividing Fences Act 1961 (WA) | Relates to the construction and repair of dividing fences between certain lands. |
| Bush Fires Act 1954 | Makes better provision for diminishing the dangers resulting from bush fires, for the prevention, control and extinguishment of bush fires. |
| Aboriginal Heritage Act 1972 | Provision for the preservation on behalf of the community of places and objects customarily used by or traditional to the original inhabitants of Australia or their descendants |
| Native Title Act (state provisions) Act 1999 | Provides for the recognition and protection of native title and to establish ways in which future dealings affecting native title may proceed. |
| Occupational Safety and Health Act 1984 (WA) | Provides for the promotion, coordination, administration and enforcement of Safety and Health in WA. Places emphasis on the prevention of accidents and injury. |
| Disability Discriminations Act 1992 | Sets out requirements to eliminate as far as possible discrimination against persons on the grounds of disability in the areas of work, accommodation, clubs and sport and in the provision of facilities, services and land |
| Disability Services Act 1993 | An Act for the establishment of the Disability Services commission and the Ministerial Advisory Council on Disability, for the furtherance of principles applicable to people with disabilities, for the funding and provision of services to such people that meet certain objectives and for the resolution of complaints by such people. |
| Australian Standards | • AS 4685.0:2017 – Playground equipment and surfacing – Development, installation, inspection, maintenance and operation • AS 4685.1-6: 2021 (6 parts) – Playground equipment – General safety requirements and test methods + Additional specific requirements for swings; slides; runways; carousels; rocking equipment • AS 4685.11:2012 – Playground equipment – Additional specific safety requirements and test methods for spatial networks • AS 4422:2016 – Playground surfacing – Specifications, requirements and test method |

## Appendix B	Major Park & Environment New Capital Works Projects

| PROJECT | ESTIMATE | YEAR |
| --- | --- | --- |
| Coolbellup Revitalisation Strategy | $250,000 | 2020/21 |
| The Lakes Revitalisation Strategy (inc South Lake Power line) | $360,000 | 2020/21 |
| Manning Park Master Plan | $693,630 | 2020/21 |
| Streetscape Improvements | $350,000 | 2020/21 |
| Playground Shade Structure (Implementation Program Strategy) | $60,000 | 2020/21 |
| Public Requests | $60,000 | 2020/21 |
| Park Lighting - Sports Oval Lighting Renewal Program | $15,000 | 2020/21 |
| Park Lighting - Public Lighting Conversion Program | $100,000 | 2020/21 |
| Undeveloped POS programs | $500,000 | 2020/21 |
| Signage Strategy | $150,000 | 2020/21 |
| Skate Park - Radiata Park | $600,000 | 2020/21 |
| Environment Development | $625,000 | 2020/21 |
| Total Year | $3,763,630 | 2020/21 |
| Total Year inc 2% CPI |  |  |
| Coolbellup Revitalisation Strategy | $250,000 | 2021/22 |
| The Lakes Revitalisation Strategy (inc South Lake Power line) | $250,000 | 2021/22 |
| Coogee Beach Master Plan | $300,000 | 2021/22 |
| Manning Park Master Plan | $687,561 | 2021/22 |
| Urban Forest Plan (UFP) | $400,000 | 2021/22 |
| Streetscape Improvements | $350,000 | 2021/22 |
| Playground Shade Structure (Implementation Program Strategy) | $60,000 | 2021/22 |
| Public Requests | $60,000 | 2021/22 |
| Park Lighting - Sports Oval Lighting Renewal Program | $175,000 | 2021/22 |
| Park Lighting - Public Lighting Conversion Program | $100,000 | 2021/22 |
| Undeveloped POS programs | $500,000 | 2021/22 |
| Street Tree & Parks Tree Database | $150,000 | 2021/22 |
| Signage Strategy | $150,000 | 2021/22 |
| NAM Strategy Review (Major) | $50,000 | 2021/22 |
| Santich Park (Carpark & Lighting) | $750,000 | 2021/22 |
| Mater Christi (Masterplan - Subject to Water) | $750,000 | 2021/22 |
| Skate Park - Nicholson Res | $600,000 | 2021/22 |
| Environment Development | $651,000 | 2021/22 |
| Total Year | $6,233,561 | 2021/22 |
| Total Year inc 2% CPI | $6,358,232 | 2021/22 |
| Coolbellup Revitalisation Strategy | $250,000 | 2022/23 |
| The Lakes Revitalisation Strategy (inc South Lake Power line) | $250,000 | 2022/23 |
| Manning Park Master Plan | $735,000 | 2022/23 |
| Smart Park Sustainability Master Plan | $440,000 | 2022/23 |
| Urban Forest Plan (UFP) | $500,000 | 2022/23 |
| Streetscape Improvements | $450,000 | 2022/23 |
| Playground Shade Structure (Implementation Program Strategy) | $60,000 | 2022/23 |
| Public Requests | $60,000 | 2022/23 |
| Park Lighting - Sports Oval Lighting Renewal Program | $375,000 | 2022/23 |
| Park Lighting - Public Lighting Conversion Program | $100,000 | 2022/23 |
| Undeveloped POS programs | $500,000 | 2022/23 |
| Street Tree & Parks Tree Database | $150,000 | 2022/23 |
| Signage Strategy | $150,000 | 2022/23 |
| POS Strategy Review (Major) | $50,000 | 2022/23 |
| Skate Park - Dixon Reserve | $400,000 | 2022/23 |
| Pump Track - TBA | $40,000 | 2022/23 |
| Environment Development | $678,000 | 2022/23 |
| Total Year | $5,188,000 | 2022/23 |
| Total Year inc 2% CPI | $5,397,595 | 2022/23 |
| Coolbellup Revitalisation Strategy | $120,000 | 2023/24 |
| The Lakes Revitalisation Strategy (inc South Lake Power line) | $300,000 | 2023/24 |
| Manning Park Master Plan | $740,250 | 2023/24 |
| Smart Park Sustainability Master Plan | $409,200 | 2023/24 |
| Yangebup & Little Rush Lake MP | $1,035,735 | 2023/24 |
| Urban Forest Plan (UFP) | $500,000 | 2023/24 |
| Streetscape Improvements | $450,000 | 2023/24 |
| Playground Shade Structure (Implementation Program Strategy) | $60,000 | 2023/24 |
| Public Requests | $60,000 | 2023/24 |
| Park Lighting - Sports Oval Audit | $50,000 | 2023/24 |
| Park Lighting - Sports Oval Lighting Renewal Program | $450,000 | 2023/24 |
| Park Lighting - Public Lighting Conversion Program | $100,000 | 2023/24 |
| Undeveloped POS programs | $500,000 | 2023/24 |
| Signage Strategy | $150,000 | 2023/24 |
| Environment Development | $706,000 | 2023/24 |
| Total Year | $5,631,185 | 2023/24 |
| Total Year inc 2% CPI | $5,975,859 | 2023/24 |
| Manning Park Master Plan | $745,500 | 2024/25 |
| Smart Park Sustainability Master Plan | $110,000 | 2024/25 |
| Yangebup & Little Rush Lake MP | $890,165 | 2024/25 |
| Streetscape Improvements | $450,000 | 2024/25 |
| Playground Shade Structure (Implementation Program Strategy) | $60,000 | 2024/25 |
| Public Requests | $60,000 | 2024/25 |
| Park Lighting - Sports Oval Lighting Renewal Program | $400,000 | 2024/25 |
| Park Lighting - Public Lighting Conversion Program | $100,000 | 2024/25 |
| Undeveloped POS programs | $500,000 | 2024/25 |
| Street Tree & Parks Tree Database | $200,000 | 2024/25 |
| Signage Strategy | $150,000 | 2024/25 |
| Environment Development | $735,000 | 2024/25 |
| Total Year | $4,400,665 | 2024/25 |
| Total Year inc 2% CPI | $4,763,421 | 2024/25 |
| Manning Park Master Plan | $735,000 | 2025/26 |
| Smart Park Sustainability Master Plan | $579,700 | 2025/26 |
| Yangebup & Little Rush Lake MP | $1,081,738 | 2025/26 |
| Streetscape Improvements | $450,000 | 2025/26 |
| Playground Shade Structure (Implementation Program Strategy) | $60,000 | 2025/26 |
| Public Requests | $60,000 | 2025/26 |
| Park Lighting - Sports Oval Lighting Renewal Program | $425,000 | 2025/26 |
| Park Lighting - Public Lighting Conversion Program | $100,000 | 2025/26 |
| Undeveloped POS programs | $500,000 | 2025/26 |
| Signage Strategy | $150,000 | 2025/26 |
| Environment Development | $765,000 | 2025/26 |
| Total Year | $4,906,438 | 2025/26 |
| Total Year inc 2% CPI | $5,417,104 | 2025/26 |
| Manning Park Master Plan | $756,000 | 2026/27 |
| Smart Park Sustainability Master Plan | $209,000 | 2026/27 |
| Yangebup & Little Rush Lake MP | $967,875 | 2026/27 |
| Streetscape Improvements | $550,000 | 2026/27 |
| Playground Shade Structure (Implementation Program Strategy) | $60,000 | 2026/27 |
| Public Requests | $60,000 | 2026/27 |
| Park Lighting - Sports Oval Lighting Renewal Program | $175,000 | 2026/27 |
| Park Lighting - Public Lighting Conversion Program | $100,000 | 2026/27 |
| Undeveloped POS programs | $500,000 | 2026/27 |
| Signage Strategy | $150,000 | 2026/27 |
| Environment Development | $780,000 | 2026/27 |
| Total Year | $4,307,875 | 2026/27 |
| Total Year inc 2% CPI | $4,851,367 | 2026/27 |
| Manning Park Master Plan | $756,000 | 2027/28 |
| Yangebup & Little Rush Lake MP | $918,279 | 2027/28 |
| Streetscape Improvements | $3,450,000 | 2027/28 |
| Playground Shade Structure (Implementation Program Strategy) | $60,000 | 2027/28 |
| Public Requests | $60,000 | 2027/28 |
| Park Lighting - Sports Oval Lighting Renewal Program | $475,000 | 2027/28 |
| Park Lighting - Public Lighting Conversion Program | $100,000 | 2027/28 |
| Coogee Golf Complex | $400,000 | 2027/28 |
| Total Year | $6,219,279 | 2027/28 |
| Total Year inc 2% CPI | $7,143,997 | 2027/28 |
| Manning Park Master Plan | $687,750 | 2028/29 |
| Yangebup & Little Rush Lake MP | $927,608 | 2028/29 |
| Playground Shade Structure (Implementation Program Strategy) | $60,000 | 2028/29 |
| Park Lighting - Sports Oval Lighting Renewal Program | $450,000 | 2028/29 |
| Coogee Golf Complex | $3,780,000 | 2028/29 |
| Total Year | $5,905,358 | 2028/29 |
| Total Year inc 2% CPI | $6,919,068 | 2028/29 |
| Manning Park Master Plan | $666,750 | 2029/30 |
| Yangebup & Little Rush Lake MP | $990,995 | 2029/30 |
| Playground Shade Structure (Implementation Program Strategy) | $60,000 | 2029/30 |
| Coogee Golf Complex | $4,380,575 | 2029/30 |
| Skate Park - Frankland Reserve | $400,000 | 2029/30 |
| Total Year | $6,498,320 | 2029/30 |
| Total Year inc 2% CPI | $7,766,094 | 2029/30 |

## Appendix C	Maintenance Response Levels of Service

| Asset Group | Current Response Times |
| --- | --- |
| Irrigation Infrastructure | · Damaged Sprinkler: Inspect within 3 hours, repairs completed within 2 working days |
|  | · Pump failure: Inspect within 1 working day, repairs completed within 10 working days |
|  | · Irrigation running outside standard timeframe: Inspect & repair within 4 hours |
|  | · High Priority: Inspect within 2 working days, repairs completed within 5 working days |
|  | · Low Priority: Inspect within 5 working days, repairs completed within 10 working days |
|  | · Graffiti: Inspect within 4 hours, works completed within 5 working days (if obscene works completed within 1 day) |
|  | · All customer requests inspected within 5 working days, repairs completed within 10 working days |
| Sporting Facility Infrastructure(including lighting) | · Emergency: Inspect within 3 hours, repairs completed within 10 working days |
|  | · High Priority: Inspect within 1 working day, repairs completed within 10 working days |
|  | · Low Priority: Inspect within 3 working days, repairs completed within 20 working days |
|  | ·Graffiti: Inspect within 4 hours, works completed within 5 working days (if obscene works completed within 1 day) |
|  | · All customer requests inspected within 3 working days, repairs completed within 10 working days |
| Playgrounds | · Emergency: Inspect within 3 hours, repairs completed within 5 working days |
|  | · High Priority: Inspect within 1 working day, repairs completed within 10 working days |
|  | · Low Priority: Inspect within 3 working days, repairs completed within 10 working days |
|  | · Graffiti: Inspect within 4 hours, works completed within 5 working days (if obscene works completed within 1 day) |
|  | · All customer requests inspected within 5 working days, repairs completed within 20 working days |
| Park Furniture | · Emergency: Inspect within 3 hours, repairs completed within 2 working days |
|  | · High Priority: Inspect within 1 working day, repairs completed within 10 working days |
|  | · Low Priority: Inspect within 5 working days, repairs completed within 20 working days |
|  | · Graffiti: Inspect within 4 hours, works completed within 5 working days (if obscene works completed within 1 working day) |
|  | · All customer requests inspected within 5 working days, repairs completed within 10 working days |
| Artwork | · Emergency: Inspect within 2 hours, repairs attended within 2 working days |
|  | · High Priority: Inspect within 1 day, repairs attended within 10 working days |
|  | · Low Priority: Inspect within 5 working days, repairs attended within 20 working days |
|  | · Graffiti: Inspect within 2 hours, works attended within 5 working days (if obscene works completed within 1 day) |
|  | · All customer requests inspected within 5 working days, repairs attended within 10 working days. |
|  | · Timeframes could increase due to specialist materials/techniques, artist consultation and insurance claims |
| Fencing / Walls | · Emergency: Inspect within 4 hours, repairs completed within 5 working days |
|  | · High Priority: Inspect within 1 working day, repairs completed within 10 working days |
|  | · Low Priority: Inspect within 3 working days, repairs completed within 20 working days |
|  | · Graffiti: Inspect within 4 hours, works completed within 5 working days (if obscene works completed within 1 day) |
|  | · All customer requests inspected within 5 working days, works completed within 10 working days |
| Playing Surface | · Emergency: Inspected within 4 hours, works completed within 1 working day |
|  | · High Priority: Inspected within 1 working day, works completed within 3 working days |
|  | · Low Priority: Inspected within 5 working days works completed within 10 working days |
|  | · All customer requests inspected within 5 working days, repairs completed within 10 working days |
| Trees | · Trees across roads, footpaths: Inspected within 2 hours, works completed within 12 hours |
|  | · Tree / limb fallen but safe: Inspected within 1 working day, works completed within 5 working days |
|  | · Pruning from powerlines or off property: Inspected within 5 working days, works completed within 30 working days |
|  | · All customer requests inspected within 5 days, works completed within 10 days |
| Streetscapes – Garden Beds | · Damaged by vehicle: Inspected within 5 working days, works completed within 5 working days |
|  | · All customer requests inspected within 5 working days, repairs completed within 10 working days |
| Streetscapes – Verge Mowing | · Visions hazard for Traffic: Inspected within 5 working days, works completed within 5 working days |
|  | ·Firebreak notice: Inspected within 10 working days, works completed within 10 working days |
|  | ·Unkempt verges: Inspect within 10 working days, works completed within 30 working days |
|  | · All customer requests inspected within 5 working days, works completed within 15 working days |
| Bushland | · Declared Weed: Inspect within 2 working days, remove within 10 working days |
|  | · Firebreak notice: Inspected within 5 days, works completed within 15 - 20 days |
|  | · Graffiti: Inspect within 24 hours, works completed within 5 days (if obscene works completed within 3 day) |
|  | · Damaged Footpath: Inspect within 5 days, works completed within 20 days |
|  | · Long grass: Inspect within 5 days, works completed within 10 days |
|  | · Damaged Infrastructure: Inspect within 3 days, works completed within 20 days |
|  | · All customer requests inspected within 5 - 10 working days, works completed within 10 - 90 days |
| Skate parks and Pump Tracks | · Emergency: Inspect within 3 hours, repairs completed within 3 working days |
|  | · High Priority: Inspect within 1 working day, repairs completed within 5 working days |
|  | · Low Priority: Inspect within 3 working days, repairs completed within 10 working days |
|  | · Graffiti: Inspect within 4 hours, works completed within 5 working days (if obscene works completed within 1 day) |
|  | · All customer requests inspected within 5 working days, repairs completed within 10 working days |

## Appendix D	Preliminary 10 Year Playground Renewal Program

| Reserve name | Suburb | Renewal year | CRC |
| --- | --- | --- | --- |
| Ferres Reserve | Bibra Lake | 2020/21 | $19,467 |
| Lakeridge Park | Cockburn Central | 2020/21 | $50,667 |
| Rotary Park | Coogee | 2020/21 | $47,467 |
| Jakob Reserve | Hamilton Hill | 2020/21 | $22,933 |
| Duggan Park | Hammond Park | 2020/21 | $33,333 |
| Allamanda Park | South Lake | 2020/21 | $29,600 |
| Anning Park 1 | South Lake | 2020/21 | $29,600 |
| Richmond Park | Success | 2020/21 | $22,400 |
| Levi Park | Yangebup | 2020/21 | $36,267 |
|  | Total Year 2020/21 |  | $291,734 |
| Carlhausen Park | Atwell | 2021/22 | $29,733 |
| Chorus Park | Atwell | 2021/22 | $52,000 |
| Harvest Lakes Community Centre | Atwell | 2021/22 | $18,667 |
| Kinship Park | Atwell | 2021/22 | $40,000 |
| Tranquil Park | Atwell | 2021/22 | $33,333 |
| Zodiac Park | Atwell | 2021/22 | $43,333 |
| Versailles Park | Aubin Grove | 2021/22 | $38,667 |
| Radonich Park | Beeliar | 2021/22 | $22,933 |
| Yarra Vista Park | Jandakot | 2021/22 | $45,000 |
| Mervyn Bond Park | Lake Coogee | 2021/22 | $33,333 |
| Bloodwood Park | South Lake | 2021/22 | $24,267 |
| Glen Mia Park | South Lake | 2021/22 | $29,600 |
| Sycamore Park | South Lake | 2021/22 | $33,333 |
| Wentworth Park | Success | 2021/22 | $64,133 |
| Visko Park | Yangebup | 2021/22 | $60,000 |
|  | Total Year 2021/22 |  | $568,332 |
| TOTAL YEAR inc. 2% CPI increase |  |  | $591,293 |
| Harmony P/S Oval | Atwell | 2022/23 | $51,733 |
| Camden Park | Aubin Grove | 2022/23 | $10,667 |
| Colorado Park | Aubin Grove | 2022/23 | $26,667 |
| Beeliar Community Centre | Beeliar | 2022/23 | $10,667 |
| Peregrine Park | Beeliar | 2022/23 | $47,733 |
| Malabar Park | Bibra Lake | 2022/23 | $16,267 |
| Coogee Beach Reserve | Coogee | 2022/23 | $36,267 |
| Matilda Birkett Reserve | Coolbellup | 2022/23 | $29,600 |
| Bourbon Park | Hamilton Hill | 2022/23 | $18,933 |
| Hanlon Reserve | Hamilton Hill | 2022/23 | $18,667 |
| Manning Park 1 | Hamilton Hill | 2022/23 | $116,267 |
| Manning Park 3 | Hamilton Hill | 2022/23 | $5,333 |
| Rodd Park | Hamilton Hill | 2022/23 | $18,400 |
| Botany Park | Hammond Park | 2022/23 | $74,000 |
| Blackthorne Park | South Lake | 2022/23 | $25,333 |
| Bullfinch Park | Spearwood | 2022/23 | $33,333 |
| Evelyn Massey Park | Success | 2022/23 | $42,933 |
| Milkwort Park | Success | 2022/23 | $65,867 |
| Holdsworth Reserve | Wattleup | 2022/23 | $22,933 |
| Warthwyke Park | Yangebup | 2022/23 | $29,600 |
|  | Total Year 2022/23 |  | $701,200 |
| TOTAL YEAR inc. 2% CPI increase |  |  | $744,119 |
| Bologna Park | Aubin Grove | 2023/24 | $72,533 |
| Tangle Park | Aubin Grove | 2023/24 | $36,267 |
| Bibra Lake Reserve (Progress Drive 1) | Bibra Lake | 2023/24 | $47,467 |
| Picottee Park | Coogee | 2023/24 | $29,600 |
| Coolbellup Child Care Services | Coolbellup | 2023/24 | $50,667 |
| Enright Reserve (2) Exercise Equipment Only | Hamilton Hill | 2023/24 | $28,000 |
| Arabella Park | North Coogee | 2023/24 | $66,667 |
| Lucretia Park | North Coogee | 2023/24 | $53,333 |
| Edwardes Park Forpark Set Only | Spearwood | 2023/24 | $22,933 |
| Gandossi Park | Success | 2023/24 | $2,933 |
| Magnolia Reserve | Success | 2023/24 | $33,333 |
| Success Regional Sporting Facility | Success | 2023/24 | $58,400 |
| Lopresti Park | Yangebup | 2023/24 | $36,267 |
| Perena Rocchi Reserve 2 | Yangebup | 2023/24 | $20,000 |
| Ronsard Reserve 2 | Yangebup | 2023/24 | $7,733 |
| Spinnaker Reserve | Yangebup | 2023/24 | $38,667 |
|  | Total Year 2023/24 |  | $604,800 |
| TOTAL YEAR inc. 2% CPI increase |  |  | $654,655 |
| Aubin Grove Reserve | Aubin Grove | 2024/25 | $86,667 |
| Grove Reserve | Aubin Grove | 2024/25 | $26,667 |
| Observatory Park | Aubin Grove | 2024/25 | $24,533 |
| Radiata Park | Aubin Grove | 2024/25 | $36,267 |
| Bibra Lake Reserve (Fitness) | Bibra Lake | 2024/25 | $27,067 |
| Sherbrooke Garden Reserve | Bibra Lake | 2024/25 | $28,933 |
| Coolbellup Community Hub | Coolbellup | 2024/25 | $40,000 |
| Watterton Park | Hamilton Hill | 2024/25 | $33,333 |
| Barrow Park | North Coogee | 2024/25 | $35,733 |
| Jubilee Park | Success | 2024/25 | $42,667 |
| Success Reserve Creche | Success | 2024/25 | $33,333 |
| Minori Reserve | Yangebup | 2024/25 | $62,533 |
|  | Total Year 2024/25 |  | $477,733 |
| TOTAL YEAR inc. 2% CPI increase |  |  | $527,456 |
| Beeliar Reserve | Beeliar | 2025/26 | $52,267 |
| Bibra Lake Community Centre | Bibra Lake | 2025/26 | $33,333 |
| Windmill Reserve | Bibra Lake | 2025/26 | $22,933 |
| Hargreaves Park 2 | Coolbellup | 2025/26 | $22,933 |
| Davilak Oval 1 | Hamilton Hill | 2025/26 | $22,933 |
| Hobbs Park | Hamilton Hill | 2025/26 | $29,600 |
| Isted Reserve | Hamilton Hill | 2025/26 | $29,600 |
| Manning Park 4 | Hamilton Hill | 2025/26 | $20,000 |
| Hagan Park | Lake Coogee | 2025/26 | $36,133 |
| Kevin Bowman Reserve | Lake Coogee | 2025/26 | $45,867 |
| Bassett Reserve | North Lake | 2025/26 | $27,200 |
| Monaco Park | North Lake | 2025/26 | $16,267 |
| Market Garden Swamp North 1 | Spearwood | 2025/26 | $38,933 |
| Horse Hire Park | Beeliar | 2025/26 | $49,200 |
| Milgun Reserve 1 | Yangebup | 2025/26 | $40,000 |
| Milgun Reserve 2 | Yangebup | 2025/26 | $15,200 |
|  | Total Year 2025/26 |  | $502,399 |
| TOTAL YEAR inc. 2% CPI increase |  |  | $565,783 |
| Princeton Park | Aubin Grove | 2026/27 | $70,000 |
| Meares Park | Bibra Lake | 2026/27 | $47,333 |
| The Glade | Bibra Lake | 2026/27 | $36,267 |
| Doherty Reserve | Coolbellup | 2026/27 | $25,867 |
| Rinaldo Reserve | Coolbellup | 2026/27 | $67,200 |
| Piesley Park | Hammond Park | 2026/27 | $42,133 |
| Solta Park | Lake Coogee | 2026/27 | $62,533 |
| Bavich Park | Spearwood | 2026/27 | $64,667 |
| Bishop Park | Spearwood | 2026/27 | $39,200 |
| Condil Park | Success | 2026/27 | $31,200 |
| Dunraven Reserve | Yangebup | 2026/27 | $47,867 |
|  | Total Year 2026/27 |  | $534,267 |
| TOTAL YEAR inc. 2% CPI increase |  |  | $613,705 |
| Powell Reserve | Coogee | 2027/28 | $55,200 |
| De Marchi Park | Coolbellup | 2027/28 | $44,933 |
| Mamillius Park | Coolbellup | 2027/28 | $22,933 |
| Rinaldo Place Reserve | Coolbellup | 2027/28 | $51,200 |
| Tempest Park | Coolbellup | 2027/28 | $52,533 |
| Wheeler Reserve | Hamilton Hill | 2027/28 | $49,067 |
| Weetman | Hammond Park | 2027/28 | $20,000 |
| Albion Park | Lake Coogee | 2027/28 | $105,867 |
|  | Total Year 2027/28 |  | $401,733 |
| TOTAL YEAR inc. 2% CPI increase |  |  | $470,694 |
| Atwell Oval | Atwell | 2028/29 | $122,133 |
| Harvest Lakes Community Centre | Atwell | 2028/29 | $10,667 |
| Mariposa Park | Beeliar | 2028/29 | $25,600 |
| Crown Park | Coogee | 2028/29 | $40,000 |
| Mills St Reserve | Coogee | 2028/29 | $23,333 |
| Coolbellup Library | Coolbellup | 2028/29 | $53,333 |
| Robb Park 2 | Coolbellup | 2028/29 | $7,600 |
| Enright Reserve (1) | Hamilton Hill | 2028/29 | $46,667 |
| Enright Reserve (2) Exercise Equipment Only | Hamilton Hill | 2028/29 | $53,333 |
| Serventy Park | Hammond Park | 2028/29 | $22,933 |
| Bunker Gardens | Jandakot | 2028/29 | $22,400 |
| Wineberry Park | South Lake | 2028/29 | $36,267 |
| Beale Park | Spearwood | 2028/29 | $42,933 |
| Smart Park 2 | Spearwood | 2028/29 | $38,000 |
|  | Total Year 2028/29 |  | $545,199 |
| TOTAL YEAR inc. 2% CPI increase |  |  | $651,563 |
| Atwell Community Centre | Atwell | 2029/30 | $87,067 |
| Atwell Community Centre (Fenced) | Atwell | 2029/30 | $18,000 |
| Atwell Reserve (Tapper Reserve) | Atwell | 2029/30 | $81,600 |
| Hakea Park | Beeliar | 2029/30 | $87,867 |
| Meller Park | Bibra Lake | 2029/30 | $89,467 |
| Robb Park 1 | Coolbellup | 2029/30 | $94,267 |
| Anning Park 1 | South Lake | 2029/30 | $49,600 |
|  | Total Year 2029/30 |  | $507,868 |
| TOTAL YEAR inc. 2% CPI increase |  |  | $619,088 |

## Appendix E	Preliminary 6 Year Irrigation Renewal Program

| Location | Suburb | 2020/21 | 2021/22 | 2022/23 | 2023/24 | 2024/25 | 2025/26 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Anning Park | South Lake |  | $ 250,000 |  |  |  |  |
| Banjup Community | Banjup |  |  |  |  |  |  |
| Blackburn Reserve | South Lake |  |  |  |  |  |  |
| Brandwood Reserve | Leeming |  |  |  |  |  |  |
| Cabinet Renewal | Not Applicable | $ 17,562 | $ 150,000 | $ 150,000 | $ 150,000 | $ 150,000 | $ 150,000 |
| Central Control | Not Applicable | $ 19,015 | $ 85,000 | $ 85,000 | $ 85,000 | $ 85,000 | $ 100,000 |
| Classon Reserve | Leeming |  |  |  |  |  |  |
| Coogee Beach Reserve | Coogee |  |  |  |  |  |  |
| Dixon Reserve | Hamilton Hill |  |  |  |  |  |  |
| Dubove Park | Spearwood | $ 45,000 |  |  |  | $ 100,000 |  |
| Fairway Park | Jandakot |  |  | $ 35,000 |  |  |  |
| Goodchild Park | Hamilton Hill |  |  |  | $ 170,000 |  |  |
| Habitat Reserve | Beeliar |  |  |  |  |  |  |
| Heatherlea Reserve | Leeming |  |  |  |  |  |  |
| Hobbs Park | Hamilton Hill |  |  |  |  |  |  |
| Isted Reserve | Hamilton Hill |  |  |  |  |  |  |
| Jackob Reserve | Hamilton Hill |  |  |  |  |  |  |
| Jakovich Reserve | Atwell |  |  |  |  |  |  |
| Jan Hammond Park | Success |  |  |  | $ 120,000 |  |  |
| Jubilee Park | Success |  |  |  |  |  |  |
| Kurrajong Reserve | Atwell |  |  |  |  |  |  |
| Len Packham Reserve | Coolbellup |  |  |  |  |  |  |
| Lucius Park | Spearwood |  |  | $ 80,000 |  |  | $ 140,000 |
| Mannning Park | Hamilton Hill |  |  |  |  |  |  |
| Meares Park | Bibra lake |  |  |  |  |  |  |
| Nicholson Reserve | Yangebup |  |  |  |  |  | $ 150,000 |
| Perena Rocchi Reserve | Yangebup |  |  |  |  |  |  |
| Poole Reserve | Coogee | $ 114,000 |  |  |  |  |  |
| Pump Renewal Allocation | Not Applicable | $ 300,000 | $ 200,000 | $ 200,000 | $ 200,000 | $ 200,000 | $ 200,000 |
| Rotary Park | Coogee |  | $ 40,000 |  |  |  | $ 75,000 |
| Santich Park | Munster |  |  |  |  |  |  |
| South Coogee Reserve | Munster |  |  |  |  | $ 120,000 |  |
| Southwell Community Centre | Hamilton Hill |  |  |  |  |  |  |
| Steiner Park | Success |  | $ 65,000 |  |  |  |  |
| Turnbury Park | Jandakot |  |  |  |  |  |  |
| Visko Park | Yangebup |  |  |  |  |  |  |
| Warthwyke Park | Yangebup |  |  | $ 40,000 |  |  |  |
| Yarra Vista Park | Jandakot |  |  |  |  |  |  |
| Total per Year |  | $ 495,577 | $ 790,000 | $ 590,000 | $ 725,000 | $ 655,000 | $ 815,000 |
| TOTAL PER YEAR inc. 2 % CPI |  | $ 505,489 | $ 821,916 | $ 626,113 | $ 784,763 | $ 723,173 | $ 917,822 |

## Appendix F Projected 10 Year Infrastructure Renewal Program

| PARK NAME | Park Class | 20/21 | 21/22 | 22/23 | 23/24 | 24/25 | 25/26 | 26/27 | 27/28 | 28/29 | 29/30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BANKSIA EUCALYPT WOODLAND PARK | BUSHLAND |  | $2,047 |  |  | $3,500 |  | $10,550 | $170 |  |  |
| BEELIAR RESERVE | BUSHLAND |  |  |  |  |  |  | $49 |  |  |  |
| BEELIAR RESERVE | BUSHLAND |  |  |  |  |  |  | $3,500 |  |  |  |
| BIBRA LAKE RESERVE | BUSHLAND | $21,049 | $6,500 | $3,500 | $1,133 | $76,689 | $4,500 | $38,549 | $7,000 | $27,049 |  |
| BIBRA LAKE RESERVE | BUSHLAND |  |  |  |  |  |  | $50 |  |  |  |
| BOODJAR MOOLINY RESERVE | BUSHLAND |  |  |  |  | $3,500 |  | $7,000 |  |  |  |
| BOSWORTH RESERVE | BUSHLAND |  |  |  |  |  |  |  | $28,196 |  |  |
| BRANDWOOD RESERVE | BUSHLAND |  |  |  |  | $12,500 |  |  | $500 |  |  |
| BUCKINGHAM RESERVE | BUSHLAND |  | $170 |  |  |  |  |  |  |  |  |
| C Y O'CONNOR RESERVE | BUSHLAND | $3,500 |  |  |  | $10,500 | $4,500 |  |  |  |  |
| C Y O'CONNOR RESERVE | BUSHLAND | $7,000 |  | $3,608 |  |  |  |  |  |  |  |
| CHRISTMAS TREE PARK | BUSHLAND |  |  |  |  | $3,500 |  |  |  |  |  |
| CLEMENTINE PARK | BUSHLAND |  |  |  |  |  |  | $13,049 |  | $16,500 |  |
| COCOS PARK RESERVE | BUSHLAND |  |  |  |  |  |  |  | $170 |  |  |
| COOGEE BEACH RESERVE | BUSHLAND |  | $170 | $3,500 |  | $12,500 | $9,000 | $7,000 | $4,250 | $12,000 | $18,824 |
| COOGEE BEACH RESERVE | BUSHLAND |  |  | $28,336 |  | $14,000 |  |  |  |  |  |
| DENIS DE YOUNG RESERVE | BUSHLAND |  | $6,132 | $6,608 |  | $17,357 |  |  | $66,509 |  |  |
| ECO PARK | BUSHLAND |  |  |  |  |  |  | $21,000 |  |  |  |
| EMMA TREEBY RESERVE | BUSHLAND |  |  |  |  |  |  |  | $2,000 | $5,000 |  |
| FRANKLAND PARK | BUSHLAND |  | $70,400 |  |  |  |  |  | $2,000 |  |  |
| FRANKLAND PARK | BUSHLAND |  |  |  |  | $3,047 |  |  |  |  |  |
| GENOA PARK | BUSHLAND |  |  |  |  |  |  | $21,000 |  |  |  |
| KRAEMER RESERVE | BUSHLAND |  |  |  |  |  |  |  | $340 |  |  |
| KURRAJONG PARK | BUSHLAND |  |  |  |  | $49 |  |  |  |  |  |
| LAKE COOGEE RESERVE | BUSHLAND |  |  |  |  |  |  | $14,000 | $19,729 | $14,000 |  |
| LAKE COOGEE RESERVE | BUSHLAND |  |  |  |  |  |  | $3,500 |  |  |  |
| LITTLE RUSH LAKE RESERVE | BUSHLAND |  |  |  |  | $49 |  |  |  | $1,600 | $50 |
| LUKIN SWAMP RESERVE | BUSHLAND |  |  |  |  |  |  |  | $2,000 |  |  |
| MANNING PARK | BUSHLAND | $14,000 | $8,500 | $51,920 | $31,827 | $91,163 | $13,500 | $14,098 | $5,060 | $2,500 | $50,207 |
| MANNING PARK | BUSHLAND |  |  |  |  | $49 |  |  |  |  |  |
| MARKET GARDEN SWAMP NORTH RESERVE | BUSHLAND |  |  |  |  | $3,500 |  |  |  |  | $4,500 |
| MARKET GARDEN SWAMP NORTH RESERVE | BUSHLAND |  |  |  |  |  |  | $3,549 |  |  |  |
| MATHER RESERVE | BUSHLAND |  | $170 |  |  |  |  |  | $49,200 |  |  |
| MCNEIL FIELD | BUSHLAND |  |  |  |  |  |  | $3,500 | $10,200 |  |  |
| PARCO PARK | BUSHLAND |  |  |  |  |  |  | $3,500 |  | $16,500 |  |
| REDEMPTORA RESERVE | BUSHLAND |  |  |  |  | $5,377 |  |  | $2,000 |  |  |
| ROPER RESERVE | BUSHLAND |  |  |  |  |  |  |  |  | $3,500 |  |
| ROSE SHANKS RESERVE | BUSHLAND |  |  | $2,657 |  |  |  |  | $3,877 |  |  |
| VERDE RESERVE | BUSHLAND |  |  |  |  |  |  |  | $4,768 |  |  |
| YANGEBUP LAKE RESERVE | BUSHLAND |  |  |  |  |  |  | $10,500 | $96,961 | $14,049 |  |
| AZELIA LEY GROUNDS | COMMUNIT |  | $3,668 | $1,750 |  |  |  |  | $24,349 |  |  |
| BANJUP COMMUNITY HALL | COMMUNIT |  |  |  |  |  |  |  | $170 |  |  |
| BEELIAR COMMUNITY CENTRE | COMMUNIT |  |  |  |  |  |  | $3,500 |  |  | $3,200 |
| BEELIAR RESERVE | COMMUNIT |  |  |  |  |  |  | $3,500 |  |  |  |
| CIVIC CENTRE GARDENS | COMMUNIT |  |  |  |  | $16,000 |  | $29,200 | $6,050 | $34,500 |  |
| COOLBELLUP LIBRARY | COMMUNIT |  |  |  |  |  |  |  |  | $7,000 | $3,200 |
| DENIS DE YOUNG RESERVE | COMMUNIT |  |  |  |  |  |  |  | $2,000 |  |  |
| DEPOT GARDENS/OPERATIONS CENTRE | COMMUNIT |  |  | $10,000 |  |  |  |  | $350 | $15,300 |  |
| ELINOR RESERVE | COMMUNIT |  |  |  |  |  |  | $3,500 |  |  |  |
| JAKOB RESERVE | COMMUNIT |  |  |  |  |  |  | $49 |  |  |  |
| JEAN WILLIS CENTRE | COMMUNIT |  |  |  |  |  |  |  | $350 | $3,500 |  |
| MANNING PARK | COMMUNIT |  |  |  |  | $3,500 |  |  |  | $3,500 | $13,030 |
| NAVAL BASE HOLIDAY PARK | COMMUNIT |  |  |  |  |  |  | $7,147 | $2,340 | $7,000 |  |
| SENIOR CITIZENS CENTRE | COMMUNIT |  |  |  |  |  |  |  | $2,259 |  | $8,151 |
| ALBION PARK | MINORPAS |  |  |  |  |  |  |  |  | $16,500 |  |
| ALLAMANDA PARK | MINORPAS |  |  |  |  |  |  |  |  | $6,943 |  |
| ALLENDALE PARK | MINORPAS |  |  |  |  |  |  | $3,549 | $31,450 | $7,000 |  |
| AMETHYST PARK | MINORPAS |  |  |  |  |  |  | $13,000 |  | $7,098 |  |
| ANCHORAGE PARK | MINORPAS |  |  |  |  |  |  | $7,000 |  |  |  |
| ATWELL RESERVE | MINORPAS |  |  |  |  |  |  |  | $5,170 |  |  |
| BALBOA PARK | MINORPAS |  |  |  |  | $7,000 |  | $10,549 |  |  |  |
| BARROW PARK | MINORPAS |  |  |  |  | $24,500 |  | $75,549 | $1,390 | $30,000 |  |
| BASSETT RESERVE | MINORPAS |  |  |  |  | $3,047 |  | $49 | $170 | $5,000 |  |
| BAVICH PARK | MINORPAS |  |  |  |  | $10,096 |  | $10,500 | $7,047 |  | $50 |
| BEAUMONT PARK | MINORPAS |  |  |  |  | $5,070 |  | $30,049 | $2,996 | $23,000 | $2,500 |
| BEEDELUP (NOLA WATERS) | MINORPAS |  |  |  |  |  |  |  | $3,047 |  |  |
| BINDJAR RESERVE | MINORPAS |  |  |  |  |  |  | $10,500 |  | $49 |  |
| BISHOP PARK | MINORPAS |  |  |  |  | $6,896 |  | $3,500 | $1,150 | $49 | $5,000 |
| BLACKBURN PARK | MINORPAS |  |  |  |  | $10,500 |  | $3,500 | $500 |  |  |
| BLACKTHORNE PARK | MINORPAS |  |  |  |  |  |  |  |  | $2,200 |  |
| BLOODWOOD PARK | MINORPAS |  |  |  |  |  |  | $3,500 |  | $10,549 |  |
| BLUE BOY | MINORPAS |  |  |  |  |  |  | $14,000 |  |  |  |
| BLUE SQUIL PARK | MINORPAS |  |  |  |  |  |  | $3,549 | $6,000 | $10,500 |  |
| BONDI PARK | MINORPAS |  |  |  |  | $3,500 |  | $3,500 |  |  |  |
| BORONIA PARK | MINORPAS |  |  |  |  |  |  | $7,000 |  | $7,000 |  |
| BOURBON PARK | MINORPAS |  |  |  |  | $3,500 |  | $7,000 |  |  |  |
| BOYD RESERVE | MINORPAS |  |  |  |  |  |  |  | $178 | $1,600 |  |
| BP PIPELINE | MINORPAS |  | $259 |  |  |  |  |  |  |  |  |
| BRANDWOOD RESERVE | MINORPAS |  |  |  |  |  |  | $49 | $3,047 | $7,000 |  |
| BROADWATER PARK | MINORPAS |  |  |  |  |  |  | $3,549 | $120 | $14,000 |  |
| BULLFINCH PARK | MINORPAS |  |  |  |  | $3,549 |  |  |  |  |  |
| BUNKER GARDENS | MINORPAS |  |  |  |  |  |  | $3,500 |  |  |  |
| BUSHLAND | MINORPAS |  |  |  |  |  |  |  | $2,000 |  |  |
| CAPRICORN PARK | MINORPAS |  |  |  |  |  |  |  |  | $49 |  |
| CARLHAUSEN PARK | MINORPAS |  |  | $3,500 |  |  |  |  | $2,000 | $3,549 | $4,500 |
| CELOSIA PARK | MINORPAS |  |  |  |  | $3,500 |  |  |  |  |  |
| CHORUS PARK | MINORPAS |  |  |  |  | $3,500 |  | $3,549 |  |  |  |
| CHRISTMAS TREE PARK | MINORPAS |  | $2,250 |  |  | $5,000 |  | $10,549 | $750 |  |  |
| CILANTRO PARK | MINORPAS |  |  |  |  | $49 |  | $13,300 | $1,877 | $10,500 | $4,000 |
| CLASSON PARK | MINORPAS |  |  |  |  | $7,049 |  | $49 | $7,197 |  | $5,000 |
| CLEMENTINE PARK | MINORPAS |  |  |  |  |  |  | $49 |  | $3,500 |  |
| COCKBURN CENTRAL STREETSCAPES | MINORPAS |  |  |  |  | $21,000 |  | $24,500 | $1,000 |  | $5,000 |
| COLORADO PARK | MINORPAS |  | $2,800 |  |  | $9,000 |  | $7,220 |  |  |  |
| CONDIL PARK | MINORPAS |  |  |  |  |  |  | $7,000 |  |  |  |
| CONGENIAL PARK | MINORPAS |  | $259 |  |  |  |  | $21,500 |  | $10,500 |  |
| COOGEE BEACH RESERVE | MINORPAS |  |  |  | $35,000 |  |  | $26,000 |  | $7,000 | $9,500 |
| COOGEE LAKE | MINORPAS |  |  |  |  | $340 |  |  | $13,413 |  |  |
| COSTA PARK | MINORPAS |  |  |  |  |  |  | $7,049 |  |  | $5,000 |
| COSTATA | MINORPAS |  |  |  |  |  |  |  | $170 |  |  |
| COUNSEL RESERVE | MINORPAS |  |  |  |  |  | $126,456 |  |  |  |  |
| CROWN PARK | MINORPAS |  |  |  |  |  |  | $10,500 |  | $4,000 |  |
| DALMATIA PARK | MINORPAS |  |  |  |  |  |  | $14,000 |  | $7,000 |  |
| DAVID GREGG PARK | MINORPAS |  |  |  |  |  |  | $49 |  | $12,500 | $4,500 |
| DAVIESIA PARK | MINORPAS |  |  |  |  |  |  |  |  | $7,000 |  |
| DE MARCHI PARK | MINORPAS |  | $3,047 |  |  |  |  | $3,500 |  | $1,500 |  |
| DEAN PARK | MINORPAS |  |  |  |  |  |  | $3,500 | $3,047 |  | $50 |
| DELARONDE PARK | MINORPAS |  |  |  |  |  |  | $7,000 |  |  |  |
| DELLER DRIVE POS | MINORPAS |  |  |  |  | $3,047 |  |  | $6,094 |  |  |
| DENISE OATES PARK | MINORPAS |  |  |  |  |  |  | $14,049 |  |  |  |
| DESERTPEA PARK | MINORPAS |  |  |  |  |  |  |  |  | $10,500 |  |
| DICKERSON PARK | MINORPAS |  |  |  |  |  |  | $6,100 |  | $12,100 |  |
| DIEDERICH PARK | MINORPAS |  |  |  |  |  |  | $3,500 |  |  |  |
| DITULLIO PARK | MINORPAS |  |  |  |  |  |  |  |  | $3,500 |  |
| DIXON RESERVE | MINORPAS |  |  |  |  | $13,547 |  | $10,598 | $8,500 | $12,549 |  |
| DODD RESERVE | MINORPAS |  |  |  |  | $3,500 |  | $3,500 |  | $12,102 | $1,257 |
| DOHERTY RESERVE | MINORPAS |  |  |  |  | $500 |  | $7,220 |  |  |  |
| DUFFIELD RESERVE | MINORPAS |  |  |  |  |  |  | $49 | $3,047 | $4,000 |  |
| DUGGAN PARK | MINORPAS |  |  |  |  |  |  | $21,000 | $2,500 | $11,500 | $50 |
| DURANGO PARK | MINORPAS |  |  |  |  |  |  | $10,500 |  | $7,000 |  |
| ECO PARK | MINORPAS |  |  |  |  | $3,064 |  |  | $6,128 |  |  |
| EDULIS PARK | MINORPAS |  |  |  |  | $7,000 |  | $7,049 |  |  |  |
| ELIZA PONDS | MINORPAS |  |  |  |  |  |  |  |  | $35,049 | $6,000 |
| EVELYN MASSEY PARK | MINORPAS |  |  |  |  |  |  | $7,098 | $3,700 |  | $5,000 |
| FAIRWAY PARK | MINORPAS |  |  |  |  | $3,500 |  | $49 |  |  |  |
| FERRES RESERVE | MINORPAS |  |  |  |  |  |  | $3,549 | $500 | $3,500 |  |
| FIRBANK PARK | MINORPAS |  |  |  |  |  |  |  |  | $15,000 |  |
| FLOURISH PARK | MINORPAS |  |  |  |  |  |  |  |  | $31,500 | $2,500 |
| FORTINI RESERVE | MINORPAS |  |  |  |  |  |  |  | $2,000 | $3,500 |  |
| FRESHWATER RESERVE | MINORPAS |  |  |  |  |  |  |  | $24,860 | $1,649 |  |
| GAGE PARK | MINORPAS |  |  |  |  |  |  | $13,000 | $500 | $51,500 |  |
| GALIPO PARK | MINORPAS |  |  |  |  | $500 |  |  | $1,000 | $17,500 |  |
| GANDOSSI RESERVE | MINORPAS |  |  |  |  |  |  | $3,500 |  |  |  |
| GARBIN RESERVE | MINORPAS |  |  |  |  |  |  |  | $3,306 |  |  |
| GERALD RESERVE | MINORPAS |  |  |  |  | $2,000 |  |  |  |  | $554 |
| GICHA PARK | MINORPAS |  |  |  |  |  |  | $3,500 |  |  |  |
| GLEN MIA PARK | MINORPAS |  |  |  |  | $10,500 |  | $49 | $620 | $49 | $492 |
| GOODWILL PARK | MINORPAS |  |  |  |  |  |  | $28,500 | $609 | $7,000 |  |
| GRANDPRE POS | MINORPAS |  |  |  |  |  |  |  | $609 | $49 |  |
| GRASSTREE PARK | MINORPAS |  |  |  |  |  |  | $14,000 |  |  |  |
| GREENSLADE RESERVE | MINORPAS |  |  |  |  | $500 |  |  | $3,047 |  |  |
| GROVE RESERVE | MINORPAS |  |  |  |  |  |  | $3,500 |  |  |  |
| HABITAT RESERVE | MINORPAS |  |  |  |  | $3,500 |  | $7,000 |  |  |  |
| HAGAN PARK | MINORPAS |  |  |  |  | $7,000 |  | $447 | $6,197 |  |  |
| HAKEA PARK | MINORPAS |  |  |  |  | $3,500 |  |  |  |  |  |
| HANLON PARK | MINORPAS |  |  |  |  |  |  | $7,000 | $3,047 |  |  |
| HANLON RESERVE | MINORPAS |  |  |  |  |  |  | $3,500 |  |  |  |
| HARGREAVES PARK | MINORPAS |  |  |  |  | $33,145 |  | $49 | $68,060 |  | $4,500 |
| HARMONY PARK | MINORPAS |  | $3,064 | $7,580 |  | $6,590 |  | $16,500 | $21,867 |  | $11,000 |
| HEATHERLEA RESERVE | MINORPAS |  |  |  |  | $3,500 |  |  | $3,047 | $49 |  |
| HOBBS PARK | MINORPAS |  |  |  |  | $3,500 |  |  |  | $1,500 |  |
| HOLDSWORTH RESERVE | MINORPAS |  |  |  |  |  |  |  | $170 | $2,000 |  |
| HORSE HIRE PARK | MINORPAS |  |  |  |  | $3,500 |  | $7,000 |  |  |  |
| HULJICH PARK | MINORPAS |  |  |  |  | $3,047 |  |  |  | $3,500 |  |
| INGRILLI RESERVE | MINORPAS |  |  |  |  |  |  |  | $2,217 |  |  |
| INTERIM RESERVE | MINORPAS |  |  |  |  | $500 |  |  | $3,047 | $1,549 |  |
| IRVINE PARK | MINORPAS |  |  |  |  |  |  | $49 |  |  | $4,500 |
| ISTED RESERVE | MINORPAS |  |  |  |  |  |  | $98 | $3,000 |  |  |
| JAKOB PARK | MINORPAS |  |  |  |  |  |  | $3,500 |  |  |  |
| JAMY PARK | MINORPAS |  |  |  |  |  |  |  | $8,613 |  |  |
| JAN HAMMOND PARK | MINORPAS |  |  |  |  | $14,000 |  | $19,989 | $23,138 | $3,720 | $13,440 |
| JARVIS PARK | MINORPAS |  |  |  |  | $49 |  | $7,000 | $500 | $12,500 | $50 |
| JOHNSONIA PARK | MINORPAS |  |  |  |  | $7,000 |  | $3,500 |  |  |  |
| JUBILEE PARK | MINORPAS |  |  |  |  |  |  |  | $2,500 | $7,098 |  |
| KATICH PARK | MINORPAS |  | $3,047 |  |  |  |  | $7,049 | $3,806 |  |  |
| KENNACK PARK | MINORPAS |  |  |  |  | $7,000 | $4,500 |  |  |  |  |
| KEVIN BOWMAN RESERVE | MINORPAS |  | $3,064 |  |  | $16,611 |  | $9,049 |  |  |  |
| KINSHIP PARK | MINORPAS |  |  |  |  |  |  | $10,500 | $1,259 | $3,500 | $4,000 |
| KOOBOOLONG PARK | MINORPAS |  |  |  |  |  |  | $3,500 |  | $10,500 |  |
| KOOJARRA PARK | MINORPAS |  |  |  |  |  |  |  |  | $7,000 |  |
| KURRAJONG PARK | MINORPAS |  |  |  |  | $3,064 |  | $24,549 | $4,823 |  |  |
| LAKE COOGEE RESERVE | MINORPAS |  |  |  |  | $7,000 |  | $3,500 |  | $50 |  |
| LAKERIDGE PARK | MINORPAS |  |  |  |  |  |  |  |  |  | $50 |
| LEN MCTAGGART PARK | MINORPAS |  |  |  |  | $7,500 |  | $147 | $10,429 | $7,000 |  |
| LEVI PARK | MINORPAS |  |  |  |  |  |  | $7,049 | $2,500 |  |  |
| LOMBE PARK | MINORPAS |  | $9,386 |  |  | $3,500 |  |  |  | $7,000 |  |
| LOPRESTI PARK | MINORPAS |  |  |  |  |  |  | $7,049 | $259 |  |  |
| LUCKEN RESERVE | MINORPAS |  |  |  |  | $3,500 |  | $3,500 | $120 |  |  |
| MACROZAMIA | MINORPAS |  |  |  |  |  |  | $3,500 |  | $3,500 |  |
| MAMILLIUS PARK | MINORPAS |  |  |  |  |  |  |  | $3,047 | $3,000 | $50 |
| MARIPOSA PARK | MINORPAS |  |  |  |  |  | $5,000 | $7,000 |  |  | $4,000 |
| MARKET GARDEN SWAMP 3 | MINORPAS |  |  |  |  | $12,409 |  |  | $26,403 |  |  |
| MARKET GARDEN SWAMP NORTH RESERVE | MINORPAS |  |  |  |  | $3,500 | $4,500 | $24,648 |  | $98 | $4,500 |
| MARKET GARDEN SWAMP SOUTH RESERVE | MINORPAS |  |  |  |  |  |  |  |  |  |  |
| MARQUIS EAST | MINORPAS |  |  |  |  |  |  |  |  | $7,049 |  |
| MARQUIS NORTH | MINORPAS |  |  |  |  |  |  | $3,500 |  | $3,500 |  |
| MARSHWOOD PARK | MINORPAS |  |  |  |  | $3,500 |  | $3,500 | $620 |  |  |
| MATILDA BIRKETT RESERVE | MINORPAS |  |  |  |  | $3,500 |  |  | $3,047 |  |  |
| MEARES PARK | MINORPAS |  |  |  |  | $6,594 |  | $3,500 |  | $20,500 |  |
| MERVYN BOND PARK | MINORPAS |  |  | $3,500 |  | $21,000 |  | $24,500 |  |  | $4,500 |
| MIHALJEVICH PARK | MINORPAS |  |  |  |  |  |  |  |  | $9,500 |  |
| MILGUN RESERVE | MINORPAS |  |  |  |  |  |  | $3,696 | $240 | $49 |  |
| MILKWORT PARK | MINORPAS |  | $1,500 |  |  |  |  | $17,500 | $1,500 |  |  |
| MILLS STREET PLAYGROUND | MINORPAS |  |  |  |  |  |  | $50 |  | $1,500 | $5,000 |
| MINORI PARK | MINORPAS |  |  |  |  |  |  | $49 |  |  |  |
| MONACO PARK | MINORPAS |  |  |  |  | $259 |  | $98 |  | $4,000 |  |
| MONTICOLA GARDENS | MINORPAS |  |  |  |  |  |  |  |  |  | $4,500 |
| MOSEDALE PARK | MINORPAS |  |  |  |  | $3,500 |  | $49 | $170 |  | $4,500 |
| NEILSON PARK | MINORPAS |  |  |  |  |  |  | $7,000 |  |  |  |
| OBSERVATORY PARK | MINORPAS |  |  |  |  |  |  | $14,000 | $599 |  |  |
| OLD CANTEEN PARK | MINORPAS |  |  |  |  |  |  | $14,000 |  |  |  |
| OLIVE TREE PARK | MINORPAS |  |  |  |  |  |  |  |  | $7,049 |  |
| OLIVER PARK | MINORPAS |  |  |  |  |  |  | $14,000 |  | $14,000 |  |
| PEARL FLOWER PARK | MINORPAS |  |  |  |  | $3,064 |  | $9,000 |  | $3,500 | $2,500 |
| PERDITA PARK | MINORPAS |  |  |  |  | $3,549 |  | $3,500 | $3,547 |  |  |
| PEREGRINE PARK | MINORPAS |  |  |  |  |  |  |  |  | $9,000 | $4,500 |
| PERENA ROCCHI RESERVE | MINORPAS |  | $4,500 |  |  | $3,500 |  | $10,598 | $750 | $7,098 | $6,226 |
| PICOTEE PARK | MINORPAS |  |  |  |  | $3,500 |  | $49 | $5,047 |  |  |
| PIESLEY PARK | MINORPAS |  |  |  |  |  |  |  |  |  | $4,000 |
| PIPELINE RESERVE | MINORPAS |  |  |  |  |  |  | $49 | $500 |  |  |
| PLUMOSA PARK | MINORPAS |  |  |  |  | $3,500 |  |  |  |  |  |
| POOLE RESERVE | MINORPAS |  |  |  |  | $10,759 |  | $98 | $650 | $2,549 |  |
| POWELL RESERVE | MINORPAS |  |  |  |  | $3,347 |  | $49 | $3,697 | $21,549 |  |
| PRINCETON PARK | MINORPAS |  |  |  |  | $2,000 |  | $7,000 |  | $15,000 | $50 |
| PRINSEP PARK | MINORPAS |  |  |  |  | $3,047 |  | $49 |  |  |  |
| PROVINCIAL RESERVE | MINORPAS |  |  |  |  |  |  | $49 | $259 |  |  |
| PURSLANE PARK | MINORPAS |  |  |  |  |  |  | $7,000 |  |  |  |
| RADIATA PARK | MINORPAS |  |  |  |  | $7,000 |  | $37,000 | $650 | $7,000 | $100 |
| RADONICH PARK | MINORPAS |  |  |  |  | $16,343 | $4,881 | $2,000 | $14,047 | $1,500 | $50 |
| RAMSAY PARK | MINORPAS |  |  |  |  | $49 |  | $10,549 | $3,500 | $14,000 |  |
| RAVELLO RESERVE | MINORPAS |  |  |  |  |  |  |  |  | $12,500 |  |
| REEVES PARK | MINORPAS |  |  |  |  | $7,000 |  | $20,000 |  |  | $9,500 |
| REINFORCEMENT PARK | MINORPAS |  |  |  |  |  |  | $13,000 |  |  |  |
| RICHMOND PARK | MINORPAS |  |  |  |  | $16,000 |  |  | $500 |  | $9,550 |
| RINALDO PLACE RESERVE | MINORPAS |  |  |  |  |  |  | $3,500 |  |  |  |
| RINALDO RESERVE | MINORPAS |  |  |  |  | $10,800 |  | $24,500 | $500 | $16,147 |  |
| ROBB PARK | MINORPAS |  | $650 |  |  |  |  | $10,500 | $3,697 |  |  |
| RODD PARK | MINORPAS |  |  |  |  | $451 |  |  | $2,700 |  |  |
| ROLLINSON PARK | MINORPAS |  |  |  |  |  |  | $7,000 |  | $3,500 |  |
| RONSARD PARK | MINORPAS |  |  |  |  | $14,049 |  |  | $3,000 |  | $4,550 |
| ROSCOE PARK | MINORPAS |  |  |  |  | $17,500 |  | $4,100 | $3,064 |  |  |
| ROTARY PARK | MINORPAS |  |  |  |  | $12,500 |  | $49 | $7,935 | $7,000 | $5,000 |
| RUSH PARK | MINORPAS |  |  |  |  |  |  | $7,000 | $259 |  |  |
| SANTORINI PARK | MINORPAS |  |  |  |  |  |  |  |  | $7,000 |  |
| SAPPHIRE PARK | MINORPAS |  |  |  |  |  |  |  |  | $15,000 |  |
| SAWLE RESERVE | MINORPAS |  |  |  |  |  |  | $3,500 |  |  |  |
| SERVENTY PARK | MINORPAS |  |  |  |  |  |  | $7,049 |  | $3,549 |  |
| SMART PARK | MINORPAS |  |  | $3,500 |  | $13,241 |  |  | $9,050 |  | $50 |
| SOLACE PARK | MINORPAS |  |  |  |  |  |  | $14,000 |  | $3,500 |  |
| SOLTA PARK | MINORPAS |  |  |  |  |  |  |  | $9,405 |  |  |
| SOUTHWELL PARK | MINORPAS |  |  |  |  | $3,549 |  | $3,549 | $4,000 |  |  |
| SPINNAKER PARK | MINORPAS |  |  |  |  |  |  | $49 |  |  |  |
| STEINER PARK | MINORPAS |  |  |  |  |  |  |  |  | $3,549 |  |
| SUNSTONE PARK | MINORPAS |  |  |  |  |  |  | $26,000 |  |  |  |
| SYCAMORE PARK | MINORPAS |  |  |  |  | $3,500 |  | $49 |  |  | $50 |
| TANGLE PARK | MINORPAS |  |  |  |  |  |  | $23,049 |  | $49 |  |
| TAPPER PARK | MINORPAS |  |  |  |  | $23,000 | $9,500 | $7,049 | $2,500 |  | $13,600 |
| TERAMO PARK | MINORPAS |  |  |  |  |  |  | $10,500 |  |  |  |
| THE GLADE | MINORPAS |  |  |  |  |  |  | $7,049 | $868 | $49 | $50 |
| THE PLAYGROUND | MINORPAS |  |  |  |  |  |  | $25,949 |  |  |  |
| THE SIDING | MINORPAS |  |  |  |  |  |  | $3,500 |  |  |  |
| TOUCHELL PARK | MINORPAS |  |  |  |  |  |  | $3,500 |  | $3,500 | $5,000 |
| TOZER PARK | MINORPAS |  |  |  |  | $3,500 | $4,500 | $7,000 |  |  |  |
| TRANQUIL PARK | MINORPAS |  |  |  |  |  |  | $10,500 |  |  |  |
| TURNBURY PARK | MINORPAS |  |  |  |  |  |  | $9,000 |  | $10,500 |  |
| VELA LUKA PARK | MINORPAS |  |  |  |  | $10,047 |  |  |  |  |  |
| VERSAILLES PARK | MINORPAS |  |  |  |  |  |  |  |  | $14,884 | $9,000 |
| VOYAGEURS PARK | MINORPAS |  |  |  |  |  |  |  |  | $3,520 |  |
| WAMBIRI PARK | MINORPAS |  |  |  |  |  |  | $3,500 |  |  |  |
| WARTHWYKE PARK | MINORPAS |  | $3,047 |  |  |  |  | $49 | $3,547 | $3,500 | $4,500 |
| WATERBUTTONS PARK | MINORPAS |  |  |  |  |  |  | $3,500 | $3,700 | $7,049 |  |
| WEARNE PARK | MINORPAS |  |  |  |  |  |  | $3,500 |  | $3,500 | $4,000 |
| WEETMAN PARK | MINORPAS |  |  |  |  |  |  | $27,220 |  |  |  |
| WENTWORTH PARK | MINORPAS |  | $500 |  |  | $3,549 |  | $21,000 |  | $9,000 | $4,650 |
| WHEELER PARK | MINORPAS |  |  |  |  | $49 |  | $7,000 |  |  |  |
| WINDMILL RESERVE | MINORPAS |  |  |  |  | $49 |  |  |  | $3,500 |  |
| WINEBERRY PARK | MINORPAS |  |  |  |  |  |  |  | $3,047 | $3,500 |  |
| WOODLANDS PARK | MINORPAS |  |  |  |  |  |  | $7,000 | $3,948 | $2,800 |  |
| YARRA VISTA PARK | MINORPAS |  |  |  |  |  |  | $20,208 |  | $7,098 |  |
| ZODIAC PARK | MINORPAS |  |  |  |  |  |  | $10,500 |  |  |  |
| BIBRA LAKE ADVENTURE PARK | MAJORPAS |  |  | $3,500 |  | $28,000 |  | $12,178 |  | $17,678 | $6,674 |
| BIBRA LAKE RESERVE | MAJORPAS |  |  |  |  |  |  |  | $22,918 |  |  |
| BIBRA LAKE RESERVE | MAJORPAS | $1,750 | $170 | $8,133 |  | $7,369 | $5,000 | $49,500 | $2,510 |  | $62,499 |
| BIBRA LAKE RESERVE DRY GR | MAJORPAS |  | $14,015 |  |  | $170 |  |  | $17,381 |  |  |
| COOGEE BEACH RESERVE | MAJORPAS |  |  |  |  | $42,905 |  | $23,500 | $7,372 | $5,900 | $500 |
| CYO CONNOR | MAJORPAS |  | $50 |  |  | $41,776 | $31,500 |  | $59,798 |  | $9,000 |
| MACFAULL PARK | MAJORPAS |  |  |  |  | $7,317 |  | $48,098 | $23,392 | $3,500 | $1,053 |
| MALABAR PARK | MAJORPAS |  |  |  |  |  |  |  | $3,547 |  |  |
| ROLLINSON PARK NORTH | MAJORPAS |  |  |  |  |  |  | $21,500 |  | $14,000 |  |
| ANNING PARK | PLAYINGF |  |  |  |  | $7,000 |  | $16,098 | $2,620 | $12,500 | $20,121 |
| ATWELL RESERVE | PLAYINGF |  |  |  |  |  |  | $2,000 |  | $10,649 |  |
| AUBIN GROVE RESERVE | PLAYINGF |  |  |  |  | $14,000 |  | $7,000 | $3,047 | $3,500 |  |
| BAKER SQUARE | PLAYINGF |  |  |  |  | $3,500 |  | $23,049 | $5,759 | $3,800 | $7,931 |
| BEALE PARK | PLAYINGF |  |  |  |  | $7,000 |  | $147 | $40,240 | $5,000 | $3,550 |
| BEELIAR RESERVE | PLAYINGF |  | $2,500 |  |  |  |  |  | $16,230 |  |  |
| BIBRA LAKE GOLF COURSE | PLAYINGF |  |  |  |  |  |  |  | $3,047 |  |  |
| BOTANY PARK | PLAYINGF |  |  |  |  |  |  | $51,598 |  |  |  |
| DALMATINAC CLUB | PLAYINGF |  |  |  | $63,654 | $28,000 |  | $238,600 | $15,000 |  | $270,661 |
| DAVILAK OVAL | PLAYINGF |  |  |  |  | $170 |  | $5,049 | $56,947 | $3,500 | $51,653 |
| DEEPDENE PARK | PLAYINGF |  | $8,150 | $3,500 |  | $430 |  |  | $3,500 |  |  |
| DUBOVE PARK | PLAYINGF |  |  |  |  | $3,145 |  | $3,500 | $5,547 | $2,500 |  |
| EDWARDES PARK | PLAYINGF |  |  |  |  | $6,596 |  | $7,098 | $4,047 |  | $14,772 |
| ENRIGHT RESERVE | PLAYINGF |  |  |  |  | $147 |  | $9,000 | $59,047 |  | $12,786 |
| GOODCHILD PARK | PLAYINGF |  |  |  |  | $27,520 |  | $98 | $35,853 |  | $17,502 |
| HARMONY PARK | PLAYINGF |  | $3,064 |  |  | $3,500 |  |  | $170 |  | $2,500 |
| HOPBUSH PARK | PLAYINGF |  |  |  |  | $3,500 |  | $16,000 | $2,740 | $7,049 | $723 |
| LAKELANDS RESERVE | PLAYINGF |  |  |  |  |  |  | $7,500 | $2,500 |  | $10,879 |
| LEN PACKHAM RESERVE | PLAYINGF |  |  |  |  | $10,217 |  |  | $15,806 | $40,700 | $9,350 |
| LUCIUS PARK | PLAYINGF |  |  |  |  | $600 |  |  | $30,136 | $49 | $50 |
| MELLER PARK | PLAYINGF |  |  |  |  | $3,549 |  |  | $22,547 | $49 | $250 |
| NICHOLSON RESERVE | PLAYINGF |  |  |  |  | $14,000 |  | $3,598 | $1,665 |  |  |
| SANTICH PARK | PLAYINGF |  |  | $7,000 |  | $300 |  | $9,447 | $45,539 | $3,500 | $162 |
| SOUTH COOGEE RESERVE | PLAYINGF |  |  |  |  |  |  | $49 |  |  | $50 |
| SUCCESS RESERVE | PLAYINGF |  |  |  |  |  |  | $25,000 |  | $3,598 | $3,200 |
| TEMPEST PARK | PLAYINGF |  |  |  |  | $3,096 |  | $3,000 | $20,599 | $4,800 | $28,335 |
| VISKO PARK | PLAYINGF |  |  |  |  |  |  |  | $3,047 | $98 |  |
| WATSONS OVAL | PLAYINGF |  |  |  |  | $3,500 |  | $49 | $11,594 | $3,500 |  |
| ARABELLA PARK | SARPARK |  |  |  |  |  |  | $14,900 |  | $3,549 |  |
| BOMBAY PARK | SARPARK |  |  |  |  |  |  | $3,500 |  | $10,500 |  |
| CALEDONIA PARK | SARPARK |  |  |  |  |  |  | $14,000 | $1,000 |  |  |
| CHELYDRA PARK | SARPARK |  |  |  |  | $5,000 |  |  | $5,170 |  |  |
| CHEYLDRA PARL | SARPARK |  |  |  |  |  |  |  |  |  |  |
| COROMANDEL PARK | SARPARK |  |  |  |  |  |  | $3,500 |  | $21,000 |  |
| DJENARK BEACH | SARPARK |  |  |  |  |  |  | $40,000 |  | $10,500 |  |
| HYDRANGEA PARK | SARPARK |  |  |  |  |  |  |  |  | $26,049 |  |
| LUCRETIA PARK | SARPARK |  |  |  |  |  |  | $24,500 | $2,400 |  |  |
| MEDINA PARK | SARPARK |  |  |  |  |  |  | $7,000 |  |  |  |
| MEVE PARK | SARPARK |  |  |  |  |  |  | $3,500 |  |  |  |
| OMEO PARK | SARPARK |  |  |  |  |  |  | $14,000 | $350 |  |  |
| OMEO PARK - NORTH | SARPARK |  |  |  |  |  |  | $10,500 |  | $3,598 |  |
| PERLINTE PARK | SARPARK |  |  |  |  | $3,500 |  | $14,000 | $178 | $56,000 |  |
| SAYERS PARK | SARPARK |  |  |  |  |  |  |  |  | $22,800 |  |
| SOCRATES PARK | SARPARK |  |  |  |  |  |  |  |  | $7,000 |  |
| THE BANJUP MEMORIAL RESERVE | SARPARK |  |  |  |  |  |  |  |  | $1,500 |  |
| YANDI PARK | SARPARK |  |  |  |  |  |  | $22,260 |  | $9,500 |  |
| ATWELL STREETSCAPES | STREETSC |  |  |  |  | $3,500 |  |  | $259 |  |  |
| BEELIAR RESERVE | STREETSC |  |  |  |  |  | $4,500 | $3,500 |  | $12,500 | $5,000 |
| BERRIGAN LAKE RESERVE | STREETSC |  |  |  |  |  |  |  | $2,000 |  |  |
| BIBRA LAKE RESERVE | STREETSC |  |  | $473 |  |  |  | $3,500 |  |  |  |
| BOSWORTH RESERVE | STREETSC |  |  |  |  |  |  |  | $2,000 |  |  |
| BULLFINCH STREET MEDIAN | STREETSC |  |  |  |  | $259 |  |  |  |  |  |
| BUND MILGUN TO YANGEBUP | STREETSC |  |  |  |  |  |  |  |  |  |  |
| CHESTNUT RESERVE | STREETSC |  |  |  |  | $49 |  |  | $2,000 |  |  |
| DUNRAVEN RESERVE | STREETSC |  |  |  |  |  |  |  |  | $1,500 |  |
| GIL CHALWELL RESERVE | STREETSC |  |  |  |  |  |  |  | $2,000 |  |  |
| JAMY PARK | STREETSC |  |  |  |  |  |  |  | $2,000 |  |  |
| KATSURA | STREETSC |  |  |  |  |  |  |  | $5,355 |  |  |
| LEN PACKHAM RESERVE | STREETSC |  |  |  |  | $49 |  | $3,500 |  | $8,050 |  |
| PAUSIN RESERVE | STREETSC |  |  |  |  |  |  |  |  | $5,700 |  |
| ROTHWELL COURT | STREETSC |  |  |  |  |  |  |  | $18,870 |  |  |
| RSL MEMORIAL CARRINGTON STREET | STREETSC |  |  |  |  |  |  |  | $1,300 |  |  |
| SANTICH PARK | STREETSC |  |  |  |  |  |  |  |  |  | $50 |
| SHERBROOKE PARK | STREETSC |  |  |  |  |  |  | $3,500 |  |  |  |
| SPEARWOOD | STREETSC |  |  |  |  |  |  |  | $2,000 |  |  |
| SPILLER AND BOTTRILL PAW | STREETSC |  |  |  |  | $259 |  |  |  | $3,500 |  |
| STREETSCAPE BEENYUP | STREETSC |  |  |  |  |  |  | $3,500 |  |  |  |
| STREETSCAPE SUCCESS BEELI | STREETSC |  |  |  |  |  |  | $3,500 |  |  |  |
| STREETSCAPES MEVE | STREETSC |  | $3,000 |  |  |  |  |  |  |  |  |
| THE PLAYGROUND | STREETSC |  |  |  |  |  |  | $34,100 |  |  |  |
| VERGE MOWING - PRIORITY ONE ROADS | STREETSC |  |  | $7,216 |  | $5,500 |  | $7,000 |  |  |  |
| Verge Mowing -Rural, commercial, Industrial | STREETSC |  |  |  |  |  |  |  | $98,004 | $3,500 |  |
| VERGE MOWING STREETSCAPES | STREETSC |  |  |  |  |  |  |  | $170 |  |  |
| YANGEBUP LAKE RESERVE | STREETSC |  |  |  |  |  |  |  | $42,896 |  |  |
| TOTAL per year |  | $47,299 | $166,079 | $159,781 | $131,614 | $1,126,664 | $231,837 | $2,304,244 | $1,534,221 | $1,357,347 | $852,879 |
| TOTAL PER YEAR inc. 2% CPI Increase |  | $48,245 | $172,788 | $169,560 | $142,463 | $1,243,929 | $261,086 | $2,646,852 | $1,797,584 | $1,622,156 | $1,039,655 |

